WARNINGS, SAFETY REGULATIONS

GENERAL INFORMATION

MAIN FEATURES

TECHNICAL SPECIFICATIONS

AK-37RGL

AK-40RGL

AK-40RFL

CARRYING BAG CONTENTS

CLEANING AND CARE KIT CONTENTS

SAFETY INDICATOR

LOADING AND UNLOADING OF THE LAUNCHER

TELESCOPIC STOCK

SLING ASSEMBLY

PREPARING THE LAUNCHER FOR FIRING

SIGHTS FEATURES, AIMING

ACTIONS TO BE TAKEN IN CASE OF MISFIRE

ELIMINATION OF MISFIRE

USER LEVEL MAINTENANCE and CLEANING

USER DAILY and WEEKLY CARE FORM

SPARE PARTS CATALOG

MAIN PARTS AND ASSEMBLY OF THE WEAPON

PARTS LIST

IMPORTANT SAFETY MESSAGE

Children are attracted to and can operate firearms which can cause severe injuries or death. Prevent child access by always keeping gun locked, unloaded and separated from ammunition when not in use. If you keep a loaded firearm where a child obtains and improperly uses it, you may be fined or sent to prison.

Firearm Safety Depends on You

A gun is only as safe as the person operating it. You can never be overly careful when handling a firearm.

Carelessness is often the cause of shooting accidents, such as failing to keep the muzzle pointed in a safe direction, not being sure of your target and what is behind it, failing to properly engage the safety, leaving ammunition in the chamber or using improper loads. Since a bullet can never be called back once fired, such errors in gun handling can result in the loss of life, severe injury or property damage thus, it is crucial for your safety and the safety of those around you that you learn the principles of safe gun handling and storage before you begin to use your new firearm. Be a safe shooter - please read this instruction book thoroughly even if this is not your first firearm purchase as not all firearms are the same. The first step in being a safe shooter is to learn the rules for the safe operation and handling of firearms. There is nothing more important in gun handling than safety.

THE TEN RULES OF FİREARM SAFETY

The Ten Rules of Firearm Safety must be etched into your memory before you begin to handle firearms. These rules are intended to be followed by all persons handling firearms in the field, on the range or at home. Please read, review and understand these rules before you begin to use or even take your new firearm out of its box. Remember, firearms safety depends on you! Memorizing these safety rules will help prevent gun accidents. Please study these safety rules before handling your firearm.

Rule 1

Always Keep the Muzzle Pointed in a Safe Direction

This is the most basic and most important safety rule. A safe direction is one in which an accidental discharge will not cause injury to yourself, to others or property damage. This is particularly important when loading or unloading your firearm. Never point your gun at anything you do not intend to shoot. Treat every gun as if it were loaded at all times.

Rule 2

Firearms Should Be Unloaded When Not Actually in Use

Firearms should only be loaded when you are in the field or on the target range or shooting area, ready to shoot. When not in use, firearms and ammunition should be secured in a safe place, separate from each other, Remember to unload your firearm completely so that there is no ammunition in the chamber or magazine. Before handling this or any firearm, or handing it to someone else, visually check the chamber. Never assume a gun is unloaded even İf you were the last person to use it. Never cross a fence, climb a tree, wade through a stream or perform any awkward movement with a loaded gun. When in doubt, unload your gun! Never pull or push a loaded firearm toward yourself or another person. Never carry a loaded gun in a scabbard, a holster not being worn or a gun case - common sense prevails in gun safety!

Rule 3

Don't Completely Rely on Your Gun's Safety

Treat every gun as though it could fire at any time, even if you are not applying pressure to the trigger. The "safety" on a firearm is a mechanical device which, like any such device, can become inoperable at the worst possible time and fail to function. By mistake, you may think the safety is "ON" when it actually is not. Or you may think your gun is unloaded when there is actually a round of ammunition in it. The safety serves as a supplement to proper gun handling but cannot serve as a substitute for common sense. Never handle a gun carelessly and assume that the gun won't fire just because "the safety is on." Never touch the firearm's trigger until you are ready to shoot. Keep your fingers away from the trigger when loading or unloading. Never pull the trigger when the safety is engaged or when the safety is positioned between the "SAFE" and "FİRE" positions.

Alcohol, Drugs and Guns Don't Mix. Make No Mistake About It!

Never handle firearms after consuming alcohol or taking drugs that can affect your judgment. Shoot sober! Alcohol, certain kinds of drugs and firearms don't mix. Safe firearms handling requires alertness and concentration on one's actions. You cannot handle a firearm safely after consuming alcohol. Never consume anything that can impair your judgment or physical coordination when handling a firearm.

Rule 4

Be Sure of Your Target - And What Is Beyond It!

Once fired, a bullet (or shot charge) can never be called back; so before you shoot, know where the bullet is going and what it will strike. Be certain your shot will not injure someone or strike something beyond the target. Never fire in the direction of noise, a movement or at any object you cannot positively identify. Be aware that a .22 Short bullet can travel over 1 % miles. A centerfire cartridge, such as the .30-06, can send its bullet over 3 miles. Shotgun pellets can travel 500 yards and a shotgun slug has a range of over a half mile. Make sure your shot has a safe backstop such as a hillside. Keep in mind how far the bullet will travel if it misses your intended target. You are responsible for your actions and judgment.

Rule 5

Use the Correct Ammunition

Every firearm is designed to use a certain caliber or gauge of ammunition. İt is important that you use the correct ammunition for your firearm. Information on the correct ammunition to use with your firearm appears in the firearm's instruction manual and the manufacturer's markings on the firearm itself. Use of the wrong ammunition, improperly reloaded ammunition or corroded ammunition can result in the destruction of the firearm, serious personal injury and/or death. Form the habit of examining every round of ammunition before you put it into your gun to ensure it is of the proper gauge or caliber and that it is in good condition.

Rule 6

İf Your Gun Fails to Fire When the Trigger Is Pulled, Handle With Care. İf a cartridge or shell does not fire when the trigger is pulled, follow Commandment #1 and keep the firearm's muzzle pointed in a safe direction. Keep the muzzle down range with the action closed and wait at least 30 seconds (to ensure that the ammunition is not delayed in firing) before carefully opening the action, taking care to avoid exposure to the breech area and unloading the firearm and safely disposing of the ammunition.

Rule 7

Always Wear Eye and Ear Protection When Shooting

Exposure to shooting noise can permanently damage hearing. Flying debris, such as powder residue and ejected cartridge cases, can injure your eyes. Thus, it is only common sense to wear both eye protection (such as shooting glasses) and ear protection (such as a sound-muffling headset) whenever shooting. Also, wear eye protection when cleaning or disassembling your gun to ensure that cleaning solvent and tensioned parts (such as springs) do not come into contact with your eyes.

Rule 8

Be Sure the Barrel Is Clear of Obstructions Before Shooting. Discharging a firearm with an obstruction in the barrel can result in personal injury, property damage and/or death. Before you load your firearm, check the chamber and magazine to ascertain that no ammunition is inside. Also, check the inside of the barrel (called the "bore") to ensure it is free of obstructions. Even a small amount of mud, snow or excess lubricating oil or grease in the bore can cause excessive pressures resulting in a bulged or burst barrel which can injure or kill the shooter and bystanders. İt is a good idea to make a habit of cleaning the bore and checking for obstructions with a cleaning rod just before each shooting session. İf the noise or recoil experienced upon firing seems low or weak, or something doesn't feel "right," cease firing immediately and check to make sure that there is no obstruction in the barrel. Placing an undersized Shell or cartridge into a gun (such as a 20 gauge shell in a shotgun chambered for 12 gauge ammunition) can result in the smaller round of ammunition failing into the barrel and acting as an obstruction. When a round is subsequently fired, the barrel may burst, causing injury to the shooter and bystanders. For reference, re-read Commandment #5.

Rule 9

Do Not Alter or Modify Your Gun and Have It Serviced Regularly. Firearms are complex mechanisms that are designed to function properly in their original condition. Any alterations or changes made to a firearm after its manufacture can make the gun unsafe and will void its warranty. Do not jeopardize your safety or the safety of others by altering the trigger, mechanical safety or other mechanisms of your firearm. You should have your firearm periodically checked for proper functioning and serviced by a qualified gunsmith.

Rule 10

Learn the Mechanical and Handling Characteristics of Your Firearm. Not all firearms operate the same way. The method of carrying, handling and operating firearms varies with the mechanical characteristics of each gun. Thus, you should never handle any firearm until you become familiar with the safe handling, loading, unloading and carrying procedures for that particular firearm, as well as the rules for safe gun handling in general.

LEAD WARNING!

Discharging firearms in poorly ventilated areas, cleaning firearms or handling ammunition may result in exposure to lead and other substances known to cause birth defects, reproductive harm and other serious injuries. Have adequate ventilation at all times when shooting. Wash hands thoroughly after exposure.

Basics of Safe Gun Handling

1-	Always keep the muzzle pointed in a safe direction.

2-	Firearms should be unloaded when not actually in use.

3-	Don't totally rely on your gun's safety.

4-	Be sure of your target and what is beyond it.

5-	Use the correct ammunition for your firearm.

6-	If your gun fails to fire when the trigger is pulled, handle with care.

7-	Always wear eye and ear protection when shooting and cleaning.

8-	Be sure the barrel is clear of obstructions before shooting.

9-	Don't alter or modify your firearm and have your firearm(s) serviced regularly.

10-	Learn the mechanics and handling characteristics of the firearm you are using.

Safe gun handling depends on you! A safe shooter is a knowledgeable shooter.

WARNING! STOP SHOOTİNG IMMEDIATELY AND UNLOAD YOUR FİREARM if your firearm develops a mechanical malfunction; binding or stoppage, spitting powder/gas, a cartridge primer is punctured, a cartridge case is bulged or ruptured, or the sound on firing does not sound quite right. Do NOT try one more shot, but unload your firearm and take it and the ammunition to a qualified gunsmith. Do not assume that the firearm is empty merely because you checked the chamber. You must also check the bore for any obstruction.

WARNING! Always ensure that the ammunition you are using is clean, undamaged and of the proper caliber before loading your pistol. The manufacturer and distributor of this pistol disclaim any liability for the use of remanufactured, reloaded or hand-loaded ammunition. Protect your investment in this fine firearm by using only factory fresh, quality ammunition. The use of improper caliber or low-quality ammunition could result in injury, death and/or property damage upon firing. Remember, firearms safety depends on you! Follow all warnings provided by the manufacturer of the ammunition you are using.

LIABILITY

Our Products are classified as FIREARMS and are sold by AKDAŞ with the understanding that we will not be liable in any way for firearm malfunction, physical injury or property damage resulting in whole or in part from:

- Criminal or negligent shootings

- Improper, careless or malicious use

- Unauthorized modifications or replacement of parts

- Defective, inappropriate, hand-loaded or reloaded ammunition

- Corrosion/Maintenance neglect

- Other effects beyond our direct and immediate control

This limitation applies regardless of whether liability is based on contract, negligence or strict liability (including any failure to warn). AKDAŞ SİLAH A.Ş. In no event shall we be liable for incidental or consequential damages such as loss of use of property, business loss, loss of profits or profits.

Keep this manual next to your firearm. Make sure you understand the warnings, operating instructions and safety procedures. When loaning or selling a firearm, make sure this manual is included with the gun. You can request a copy of this manual from AKDAŞ SİLAH A.Ş.

THE SAFETY OF FIREARMS IS ENTIRELY THE RESPONSIBILITY OF THE SHOOTER. ALWAYS TREAT ALL FIREARMS AS LOADED!

IMPORTANT! READ THE SECTION AND WARNINGS IN THIS BOOKLET BEFORE USING THE FIREARM.

GENERAL INFORMATION

Congratulations on your purchase of AK-37-RGL, AK-40RGL, AK-40RFL.

With proper care and handling, it will give you long, reliable service. This anti-riot launcher has been designed to fire 37/38- or 40-mm gas cartridges to be used by law enforcement forces or military for riot control. Akdas Arms Company has analyzed superior features of the known anti-riot grenade launchers and created the ergonomic and user-friendly AK-37-RGL, AK-40RGL and AK-40RFL comprising and effective sight system.

MAIN FEATURES

- Light and ergonomic design for ease of use and transport

- Hard wear resistant, hard chrome coated barrel with mid carbon alloy that provides a long barrel life span

- Picatinny rails on top and sides that allow various additional equipment

- Ambidextrous safety slide

- Telescopic stock that can be easily detached thanks to the lever at the top of the receiver

- Capability of being carried or fired without telescopic stock in narrow places like the interior of armored vehicles

- Quick load in launcher loading both for right- or left-handed users

- Being ready to fire thanks to the double-action trigger system

- Sling can be attached on both the stock and receiver

- Design that protects shooters from the gas emerging after firing the launcher

- Ease of targeting and shooting thanks to the foldable vertical grip located under the barrel

TECHNICAL SPECIFICATIONS

Yeni Foto

| AK-37RGL |  |
| --- | --- |
| Caliber | 37/38 mm |
| Barrel Length | 305 mm (12") |
| Barrel Type | Smooth Barrel |
| Total Length (Telescopic Stock Open) | 735 mm (28,9") |
| Total Length (Telescopic Stock Closed) | 660 mm (26") |
| Weight (Unloaded) | 2300 gr |
| Action | Single break action |
| Trigger System | Double-action trigger |
| Safety | Ambidextrous slide safety |
| Sight System | Low position and 80/100/130-meter intervals |
| Shooting Mode | Single shot |
| Effective Range | 100 m |
| Trigger Pull Force | 40-60 Newton |
| Stock | Detachable telescopic |
| Shooting Velocity | 5-6 rounds/ minute |
| Barrel And Receiver Material | Barrel ST-42 Steel, Receiver 7075 Aluminum |
| Type Of Ammunition | 40 mm gas cartridges |
| Sling | NATO QD Tactical |

WARNING! It is forbidden to shoot HE ammunition and more of the same with AK-37RL/AK-40RL as these models are designed only to be used with gas cartridges.

Yeni Foto

| AK-40RGL |  |
| --- | --- |
| Caliber | 40 mm |
| Barrel Length | 305 mm (12") |
| Barrel Type | Smooth Barrel |
| Total Length (Telescopic Stock Open) | 735 mm (28,9") |
| Total Length (Telescopic Stock Closed) | 660 mm (26") |
| Weight (Unloaded) | 2300 gr |
| Action | Single break action |
| Trigger System | Double-action trigger |
| Safety | Ambidextrous slide safety |
| Sight System | Low position and 80/100/130-meter intervals |
| Shooting Mode | Single shot |
| Effective Range | 100 m |
| Trigger Pull Force | 40-60 Newton |
| Stock | Detachable telescopic |
| Shooting Velocity | 5-6 rounds/ minute |
| Barrel And Receiver Material | Barrel ST-42 Steel, Receiver 7075 Aluminum |
| Type Of Ammunition | 40 mm gas cartridges |
| Sling | NATO QD Tactical |

WARNING! It is forbidden to shoot HE ammunition and more of the same with AK-37RL/AK-40RL as these models are designed only to be used with gas cartridges.

Yeni Foto

| AK-40RFL |  |
| --- | --- |
| Caliber | 40 mm |
| Barrel Length | 305 mm (12") |
| Barrel Type | Smooth Barrel |
| Total Length (Telescopic Stock Open) | 735 mm (28,9") |
| Total Length (Telescopic Stock Closed) | 660 mm (26") |
| Weight (Unloaded) | 2300 gr |
| Action | Single break action |
| Trigger System | Double-action trigger |
| Safety | Ambidextrous slide safety |
| Sight System | Low position and 80/100/130-meter intervals |
| Shooting Mode | Single shot |
| Effective Range | 100 m |
| Trigger Pull Force | 40-60 Newton |
| Stock | Detachable telescopic |
| Shooting Velocity | 5-6 rounds/ minute |
| Barrel And Receiver Material | Barrel ST-42 Steel, Receiver 7075 Aluminum |
| Type Of Ammunition | 40 mm gas cartridges |
| Sling | NATO QD Tactical |

WARNING! It is forbidden to shoot HE ammunition and more of the same with AK-37RL/AK-40RL as these models are designed only to be used with gas cartridges.

CARRYING BAG CONTENTS

ÇANTA RESMİ

BAG CONTENT

| 1 | Carry bag |
| --- | --- |
| 2 | AK-37-RGL / AK-40RGL / AK-40RFL Launcher |
| 3 | Cleaning Kit |
| 4 | Sling |
| 5 | User Manuel |
| 6 | * Stock Assembly |

* AK-37-RGL, AK-40RGL, AK-40RFL ile birlikte satın alınması durumunda

CLEANING AND CARE KIT CONTENTS

Resmi

CLEANING KIT AND CONTENT

| 1 | Cleaning Kit | 4 | Flexible Cleaning Rod |
| --- | --- | --- | --- |
| 2 | Bronze helosonic barrel brush | 5 | Cleaning Rod (rope) |
| 3 | 100% cotton flat cleaning brush | 6 | Cleaning oil |

SAFETY OPERATIONS

Selecting the Launcher to the SAFE

Position: To place the launcher to the SAFE position, rotate the safety lever which is located on both of the left side and the right side of the of the trigger housing up to S. The safety lever's position must be at the position showed in the Picture?. This will lock the trigger and the firearm cannot be fired.

Picture ?

Selecting Safe Position

Selecting the Launcher to the FIRE

Position: To place the launcher to the FİRE position, rotate the safety lever which is lo- cated on both of the left side and the right side of the of the trigger housing up to F. The safety lever's position must be at the position showed in the Picture8. This will select the fire mode, and pulling the trigger will cause fire.

Picture ?

Selecting Fire Position

LOADING

How to load the launcher:

In order to load the launcher, push down the Release lever in Picture 9. So that barrel can open showed on Picture 10, and ammunition can be loaded Picture 11. Load the designed gas ammo and close the barrel manually. Picture 12

Picture 9 Release Lever			Picture 10 Breaking Barrel

Picture 11 Loading Ammo		Picture 12 Close the Barrel manually

UNLOADING

How to unload the launcher:

After firing the shell, push the Release Latch in Picture 9 to open the barrel, the empty case will protrude out of the chamber. Empty case can be manually pulled out.

This weapon is double triggered. So that after getting out the blind shell and putting the new ammunition, user doesn't need to cook it again.

WARNING: If you are unloading a weapon that has not been fired, avoid detonation either by catching the ejected round or by holding the weapon close to the ground to reduce the distance the round can fail.

How to Aim:

Aim the loaded launcher at the target. Hold the launcher firmly against shoulder. Fire the shell by pulling the trigger. (Max. range can be obtained by holding the launcher at 35“- 45“ angle).

WARNING: During firing, protect yourself (safety glasses, mask, etc.) against the gases that may come out of the barrel. We recommend that you should always take the wind at your back.

Leaf Sight Assembly:

With the front sight and the rear sight on the launcher, it allows you to shoot to the right distances.

There are holes on the leaf sight which allows shooting range (that is indicated on the launcher).

Top hole range 120 meter (131 YD)

Center hole range 100 meter (110 YD)

Bottom hole range is 70 meter (76 YD)

When firing with the launcher, the ammunition will reach that distance where you aim the target and the front sight through whichever hole you aim.

WARNING: Do not point the launcher to directly people and don't forget that, the ammunition thrown will move forward by jumping according to the ground on which it fell.

STOCK ASSEMBLY

In order to assemble the telescopic stock to the launcher, push downward the telescopic stock to the picatinny rail.

There is a button at the top rear of the launcher. İn order to remove the telescopic stock, user must push forward the button and push upward the telescopic stock from the picatinny rail.

To adjust the length of the telescopic stock to suitable size push the button at the bottom of the stock and move front or back

SLING ASSEMBLY

This product is compatible with NATO QD sling carrying strap. Insert the protrusion of the hanger latch into one of the holes located on both sides of the rear receiver or telescopic stock.

To remove it, you can remove it by pressing the button on the hanger latch.

PREPARING THE LAUNCHER TO SHOOT

- Interior of the barrel (bore) is checked whether it is clean and dry.

- Overall cleaning of the grenade launcher is checked.

- Ali the parts are checked to make sure they all function properly.

- Action is cocked and the trigger pull, and whether the firing pin protrudes after firing are checked,

- Barrel latches, opening/forward movement and rotation of the barrel are checked. Cocking of the action after barrel is pulled all the back until it is locked is checked.

- Loose connections may cause damage/breaking of the connection parts.

- Excessive oil on trigger group parts, barrel, and upper receiver should be avoided and cleaned using a dry cloth.

SIGHTS FEATURES, AIMING

To set the appropriate distance value after determining the target's distance, press the distance adjustment lever on the sight.  Aim the target so that the white dots on the front and rear sights are at the bottom center of the target (with the sight at the shooter's eye level) and release the trigger by turning the safety switch to the F (fire) position.  The sight has divisions of 80, 100, and 130 meters. The angle of the weapon's barrel increases as the distance increases, and decreases as the distance decreases.

ACTIONS TO BE TAKEN IN CASE OF MISFIRE

In case a jamming (due to delayed firing or non-firing), the following precautions must be taken until the grenade leave the barrel and reason of the failure must be detected:

a.	Directs the barrel to the target, and makes sure no one is at the fire line.

WARNING: Everyone around must clear off the area before attempting to extract the grenade.

b.	For one minute, the grenadier waits to open the barrel.

c.	During extraction, the grenadier must be highly careful.

d.	After grenade is removed from the chamber, it must be determined either the ammunition or the firing mechanism is faulty. Primer of the ammunition is checked whether there is mark of the firing pin. If there is no mark, it means that firing mechanism has failed. Fault withing the firing mechanism is repaired, and the grenade launcher is reloaded.

e.	If there is a mark of the firing pin on primer, the grenadier waits for one minute and shots again. When there is mark on the primer and the grenade is still not fired, the ammunition is rotated 180 degrees inside the chamber and grenade launcher Is fired once more. If the ammunition is still not fired, İt is taken carefully away to a safe zone and destroyed

FIXING MISFIRED GUNS

Fixing of the jammed guns is performed as describe in the chart below

| TYPE OF JAM/STOPPAGE | REASON FOR MISFIRE | REPAIR AND OPERATION TO BE DONE |
| --- | --- | --- |
| Misfire | - Safety is ON - Chamber is Empty - Defective ammunition - Breaking the Cock - Blocked sear or dirty firing pin | - Take a fire position - Load the gun - Reload the gun - Replaced - Cleaned |
| Cocking Failure | - Broken sear - Incorrect trigger cocking mechanism | - Replaced - Replaced |
| Failure to Feed | - Defective ammunition - Dirty firing chamber | - Replaced - Cleaned |
| Cartridge Extraction Problem | - Extractor breakage and jamming - Excessively dirty cartridge chamber | - Replaced - Cleaned |
| Safety Pin Does Not Stay in Position | -Safety lever breakage -Safety lever jamming | - Replaced - Adjusted |
| Back sight and front sight do not stop in the selected position | - Sight adjustment lock wear or breakage - Loosening or dislocation of the sight position screw - Back sight may have been damaged | - Replaced - Screw tightened or replaced - Replaced |

USER-LEVEL MAINTENANCE AND CLEANING

1-	GENERAL INFORMATION

Maintenance is all the measures taken to keep the grenade launcher always operational. These measures include the following considerations:

a.	Cleaning

b.	Inspection of parts

c.	Repair of parts

d.	Lubrication

2-	EQUIPMENT WITH CLEANİNG AND LUBRICATING AGENTS

a.	Cleaning Materials:

- Oil for cleaning interior of the barrel is used after shooting. İt also provides temporary protection against rust.

- Hot soapy water or just hot water are used when oil for cleaning inside the barrel is not available.

b.	Lubricants:

- Common lubricating oil is used to lubricate the grenade launcher for temperatures above 18°C.

- Special lubricating oil is used to lubricate the grenade launcher for temperatures below -18°C (Addinol weapon Oil W18).

- SAE 10 engine oil or castor oil can be used at war conditions when above-mentioned oils are not available. At the earliest time, grenade launchers lubricated with these oils must be cleaned and lubricated with proper oils.

c.	Protective Oils:

1.	When storing grenade launcher in crates, protective oils or grease must be used to protect metal parts.

3-	CLEANİNG THE FLARE LAUNCHER

After shooting all the residue arising from primer, powder, and any other stuff must be cleaned. Even though the rust-free structure of the primer makes the cleaning easier, cleaning must never be neglected. Indeed, rusting may occur due to the humidity arising from the primer in the long run if the residue from primers is not cleaned. The below mentioned cleaning method will remove any dirt except for the metal chips. Barrels rarely get dirty of metal chips and this is cleaned by professionals only. After firing, the grenade launcher is disassembled and cleaned as described;

1.	Inside the barrel:

(a)	Cotton brush is attached to the cleaning rod.

(b)	Cotton brush is wetted with the lubricant for barrel bore, hot soapy water or just water.

(c)	Cotton brush is inserted in and moved forward-backward a couple of times inside the barrel.

(d)	Next, wire brush is attached to the cleaning rod and again, moved forward-backward a couple of times inside barrel.

(e)	Using a dry cloth, barrel bore is cleaned. Cleaning cloth is checked every time it goes out. After making sure that no residue is left on the cloth, it means that of bore of the barrel is cleaned.

(f)	Finally, interior of the barrel is lubricated lightly inside to form a layer of oil.

2.	Firing Pin Housing:

Firing pin housing is cleaned using cloths, thin and thick wire cleaning brushes and lightly lubricated with bore cleaning oil.

3.	Other Parts:

a.	Dry cloth and a clay brush are used to clean other parts and exteriors. Ali parts are wiped with dry cloth. Metal parts are lightly lubricated with special protective oil.

b.	İn Cold weather (below 0°C): The Grenade Launcher is stored in a non-humid environment and is not over-oiled. Moisture and excess oil in the moving parts aggravate the operation of these parts or stop them completely. Grenade Launcher should be disassembled and cleaned thoroughly with a dry cloth. If necessary, dry cleaning should be done to remove excess oil. Parts showing signs of wear are wiped with a slightly dampened cloth using special oil.

- If possible, the grenade launcher is cleaned/maintained is a closed and warm place at room temperature.

- Lubricate inner surfaces, action parts like the bolt, springs and trigger group parts of the grenade launcher. On the other hand, external surface must be as dry as possible.

- When the grenade launcher is taken out of the warm place at low temperatures, it must be covered. Otherwise, a humidity layer will from on the grenade launcher due to rapid change of temperature, and this may lead to rusting.

- When the grenade launcher is carried from a cold environment to a warm place like a room or tent, it must be disassembled and dried.

- Keep the grenade launcher dry as far as possible, and apply function tests every 30 minutes due to freezing risk.

- Never leave hot guns on Ice or snow.

- Room temperature where the grenade launchers are stored should equal to outside temperature so as to avoid problems due to rapid change of temperature.

- When guns are transferred from a cold place to a hot one, they must be dehumidified and cleaned. Otherwise, frosting will occur on metal surfaces due to condensation.

c.	Hot and Humid Conditions:

- Clean and maintain the grenade launcher as necessary.

- Increase the frequency of inspection, maintenance and lubrication.

- Check interior of the grenade launcher regularly for rust and corrosion.

- Rifling of the barrel Is Important and should be taken care of. The bore must remain lightly lubricated when possible.

- Use the lubricants for maintenance more often.

- Store the grenade launcher in a closed case/box when possible.

- Avoid contact to sea/salty water. İn case of contact with salty water, disassemble

d.	Hot and Dry Air Conditions:

Dirt on/in the barrel and action parts must be daily cleaned. To prevent dirt from gathering at dirty areas, the grenade launcher must always remain dry. Muzzle, and mechanism must be protected from sand and dust storms by covering. After being exposed to dusty/sandy conditions, and sign of rust appears, the grenade launcher must be lubricated again.

- Regularly clean the grenade launcher.

- Avoid excessive lubrication as it will form oil residue on the gun.

- İnterior and action parts must be lubricated, yet outer surface should remain dry.

- Rust and corrosion should not be experienced at such condition, yet the guns should be checked from time to time.

- Store the grenade launcher in a closed case/box when possible.

- İnterior of the barrel must be cleaned from dust and sand using a cleaning rod and kept dry.

- To protect the gun from sand, it must be kept in a case or crate.

- Clean barrel and chamber every day.

- Try avoiding sand getting into the barrel.

- Drying and slightly lubricating the gun helps avoiding rust arising due to the sweating of hands and cheek.

ADVANCED LEVEL MAINTENANCE-REPAIR WARNING

- All screws in Grenade Launcher assembly are tightened with a torque of 1.0 Nm.

- Since all screws are used in the final assembly, Lock Tight (screw fixing adhesive) is used, if any of the screws need to be removed during maintenance and repair, they will be removed by heating method with a heater giving a temperature of approximately 300°C for 4-5 minutes.

- Muzzle side not directed to an unsafe target, the gun is placed on gun maintenance table or a flat surface. Avoid hitting the muzzle or sight system to the table.

- While cleaning interior of the barrel, cleaning must be done from chamber side to the muzzle, and muzzle should be hold lower than chamber side to prevent dirt and oil from reaching to loading/unloading and firing mechanisms.

- Remove any dirt from the cleaning rod before starting to dean the barrel.

- Insert the cleaning from chamber side and move it in direction of the muzzle. After cleaning rod goes out at the muzzle side, take the cloth out and remove the end piece. Next, pull the cleaning rod backward and complete the process.

- First, attach a dry and dean cloth (swab) to cleaning rod and apply a rough cleaning for the interior of barrel.

- Attach a cloth wet with maintenance oil and lubricate the interior of the barrel.

- Wait for about 15-20 minutes for the maintenance oil to soften any residue inside the barrel.

- Attach the provided bronze brush to the cleaning rod (Figure 49) and wet with oil.

- For every 3 shots, 1 cleaning with the bronze brush is necessary, and this ratio is valid cleaning.

- Next step of the cleaning goes on with the cotton brush (Figure 50).

- Attach a dry and dean cloth (swab) to the cleaning rod (provided with the cleaning/maintenance kit) and move the cleaning rod in the bore forward/backward until interior of the barrel becomes completely dean.

- To dean the chamber, attach the cleaning brush with Steel wire (thin or thick one as required), and brush it.

- Using the plastic wired cleaning brushes within the cleaning/maintenance kit, dust and dirt on outer surfaces on the gun can be cleaned.

TEMİZLEME GÖRSELLERİ

WARNING! When cleaning the inside of the barrel with a slush brush and slush cloth, never pull the rod from the end of the barrel towards the firing chamber!

WARNING! Never use force when disassembling, cleaning and assembling the weapon!

STORAGE

    - Preparation for Storage: Before your firearm is stored, it should be thoroughly cleaned and lightly oiled. A thin layer of high-quality firearm oil should be applied to all exposed surfaces. In humid conditions, ensure that cleaning cloths used are dry. After the metal parts are dried, they should not be touched with bare hands. For normal storage periods, metal parts are lightly oiled with protective oil, and for longer periods, they are oiled with grease. The firearm should never be wrapped with cloth or similar material, and no plug should be placed in the barrel, as these can cause the firearm to rust by absorbing moisture.

    - Cleaning After Long-Term Storage: A firearm taken out of storage after an extended period should be thoroughly cleaned and oiled with normal protective oil. If it is to be fired immediately, ensure that the oil used for storage (protective or grease) is completely removed, the inside of the barrel is dry, and there is no obstruction before firing.

BEFORE STORING, CHECK IF YOUR SEMI-AUTOMATIC PISTOL IS LOADED. STORE FIREARMS AND AMMUNITION SEPARATELY AND IN A SECURE PLACE INACCESSIBLE TO UNAUTHORIZED INDIVIDUALS. ALWAYS LOCK YOUR FIREARMS SAFELY WHEN NOT IN USE. CONSULT YOUR COUNTRY'S FIREARMS REGULATIONS FOR THE SAFE STORAGE OF FIREARMS AND AMMUNITION.

WARNING! Modifying or changing parts and/or internal safeties is dangerous and voids the warranty. This firearm is designed to function properly with original parts. It is your responsibility to ensure that all purchased parts are intended for this firearm, installed correctly, and both spare and original parts are not replaced or modified. Your firearm is a complex and delicate tool with many parts that need to be correctly associated with other parts for proper and safe operation. Assembling a firearm with incorrect or modified parts can lead to damage, danger, and malfunction, resulting in injury or death to yourself and others. Always consult a qualified gunsmith for any work on your firearm.

| USER DAILY AND WEEKLY CARE FORM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PLACES TO MAINTAIN | DAILY CARE |  |  | WEEKLY CARE |
|  |  | SERVICE BEFORE | SERVICE DURING | AFTER SERVICE |  |
| 1 | GENERAL STATUS Checking for broken, detached and non-functioning parts in the external appearance and parts of the weapon. Non-original screws and pins are replaced immediately. | X | X | X | X |
| 2 | BARREL and SHOOTING BED Checking the inside of the barrel and the firing chamber for foreign matter, scratches, broken sets and pins and needles. Cleaning and drying the inside of the barrel. | X | X | X | X |
| 3 | IGNITION SYSTEM a. Checking that the firing pin is working, cleaning and lightly lubricating the moving parts. b. Cleaning and lubricating all parts of the trigger assembly and checking the operability of each part. | X |  | X | X |
| 4 | DETERMINATION OF BARREL LIFE SPAN The barrel of the weapon is used to determine whether the barrel has reached the end of its service life. | X |  | X | X |
| INSTRUCTIONS FOR USE |  |  |  |  |  |
| 1. This form is used by the user for daily and weekly maintenance. 2. The user of the weapon is responsible for the proper use of the form, as well as the unit commanders responsible for its control and supervision. 3. The form is used as a guide for daily and weekly maintenance and is not filled in. 4. The form is also used for technical inspections and examinations. 5. The points to be maintained are marked with an X on the form. 6. The issues identified are reported to the KS/TK Commander. 7. The relevant technical instruction, technical publication and maintenance manual shall be taken as basis for the inspection, control and maintenance to be performed. |  |  |  |  |  |

SPARE PARTS CATALOG

MAIN PARTS AND ASSEMBLY OF THE WEAPON

PARTS LIST