WARNINGS, SAFETY RULES

GENERAL INFORMATION

BASIC FEATURES

TECHNICAL SPECIFICATION

AK-37UBFL

AK-40UBFL

LAYOUT OF THE GRENADE LAUNCHER İN THE CARRY BAG

CLEANING KIT AND ITS CONTENTS

ATTACHING THE GRENADE LAUNCHER TO THE INFANTRY RIFLE

DISMANTLING OF THE GRENADE LAUNCHER FROM THE INFANTRY RIFLE

ATTACHING THE MECHANICAL SIGHT TO THE LAUNCHER

TELESCOPIC STOCK AND SLING

LOADING THE GRENADE LAUNCHER

UNLOADING THE GRENADE LAUNCHER

PREPARING THE LAUNCHER TO FIRE

AIMING AND SHOOTING WITH THE GRENADE LAUNCHER

SHOOTING POSITİONS

ACTION IN CASE OF MISFIRE

REMOVAL OF MISFIRE

USER LEVEL MAINTENANCE AND CLEANING

ADVANCED MAINTENANCE-REPAIR WARNINGS

STORAGE

USER DAILY and WEEKLY MAINTENANCE FORM

SPARE PARTS CATALOGUE

Main Parts and Assembly of the Gun

Part Names

IMPORTANT SAFETY MESSAGE

Children are attracted to and can operate firearms which can cause severe injuries or death. Prevent child access by always keeping gun locked, unloaded and separated from ammunition when not in use. If you keep a loaded firearm where a child obtains and improperly uses it, you may be fined or sent to prison.

Firearm Safety Depends on You

A gun is only as safe as the person operating it. You can never be overly careful when handling a firearm.

Carelessness is often the cause of shooting accidents, such as failing to keep the muzzle pointed in a safe direction, not being sure of your target and what is behind it, failing to properly engage the safety, leaving ammunition in the chamber or using improper loads. Since a bullet can never be called back once fired, such errors in gun handling can result in the loss of life, severe injury or property damage thus, it is crucial for your safety and the safety of those around you that you learn the principles of safe gun handling and storage before you begin to use your new firearm. Be a safe shooter - please read this instruction book thoroughly even if this is not your first firearm purchase as not all firearms are the same. The first step in being a safe shooter is to learn the rules for the safe operation and handling of firearms. There is nothing more important in gun handling than safety.

THE TEN RULES OF FİREARM SAFETY

The Ten Rules of Firearm Safety must be etched into your memory before you begin to handle firearms. These rules are intended to be followed by all persons handling firearms in the field, on the range or at home. Please read, review and understand these rules before you begin to use or even take your new firearm out of its box. Remember, firearms safety depends on you! Memorizing these safety rules will help prevent gun accidents. Please study these safety rules before handling your firearm.

Rule 1

Always Keep the Muzzle Pointed in a Safe Direction

This is the most basic and most important safety rule. A safe direction is one in which an accidental discharge will not cause injury to yourself, to others or property damage. This is particularly important when loading or unloading your firearm. Never point your gun at anything you do not intend to shoot. Treat every gun as if it were loaded at all times.

Rule 2

Firearms Should Be Unloaded When Not Actually in Use

Firearms should only be loaded when you are in the field or on the target range or shooting area, ready to shoot. When not in use, firearms and ammunition should be secured in a safe place, separate from each other, Remember to unload your firearm completely so that there is no ammunition in the chamber or magazine. Before handling this or any firearm, or handing it to someone else, visually check the chamber. Never assume a gun is unloaded even İf you were the last person to use it. Never cross a fence, climb a tree, wade through a stream or perform any awkward movement with a loaded gun. When in doubt, unload your gun! Never pull or push a loaded firearm toward yourself or another person. Never carry a loaded gun in a scabbard, a holster not being worn or a gun case - common sense prevails in gun safety!

Rule 3

Don't Completely Rely on Your Gun's Safety

Treat every gun as though it could fire at any time, even if you are not applying pressure to the trigger. The "safety" on a firearm is a mechanical device which, like any such device, can become inoperable at the worst possible time and fail to function. By mistake, you may think the safety is "ON" when it actually is not. Or you may think your gun is unloaded when there is actually a round of ammunition in it. The safety serves as a supplement to proper gun handling but cannot serve as a substitute for common sense. Never handle a gun carelessly and assume that the gun won't fire just because "the safety is on." Never touch the firearm's trigger until you are ready to shoot. Keep your fingers away from the trigger when loading or unloading. Never pull the trigger when the safety is engaged or when the safety is positioned between the "SAFE" and "FİRE" positions.

Alcohol, Drugs and Guns Don't Mix. Make No Mistake About It!

Never handle firearms after consuming alcohol or taking drugs that can affect your judgment. Shoot sober! Alcohol, certain kinds of drugs and firearms don't mix. Safe firearms handling requires alertness and concentration on one's actions. You cannot handle a firearm safely after consuming alcohol. Never consume anything that can impair your judgment or physical coordination when handling a firearm.

Rule 4

Be Sure of Your Target - And What Is Beyond It!

Once fired, a bullet (or shot charge) can never be called back; so before you shoot, know where the bullet is going and what it will strike. Be certain your shot will not injure someone or strike something beyond the target. Never fire in the direction of noise, a movement or at any object you cannot positively identify. Be aware that a .22 Short bullet can travel over 1 % miles. A centerfire cartridge, such as the .30-06, can send its bullet over 3 miles. Shotgun pellets can travel 500 yards and a shotgun slug has a range of over a half mile. Make sure your shot has a safe backstop such as a hillside. Keep in mind how far the bullet will travel if it misses your intended target. You are responsible for your actions and judgment.

Rule 5

Use the Correct Ammunition

Every firearm is designed to use a certain caliber or gauge of ammunition. İt is important that you use the correct ammunition for your firearm. Information on the correct ammunition to use with your firearm appears in the firearm's instruction manual and the manufacturer's markings on the firearm itself. Use of the wrong ammunition, improperly reloaded ammunition or corroded ammunition can result in the destruction of the firearm, serious personal injury and/or death. Form the habit of examining every round of ammunition before you put it into your gun to ensure it is of the proper gauge or caliber and that it is in good condition.

Rule 6

İf Your Gun Fails to Fire When the Trigger Is Pulled, Handle With Care. İf a cartridge or shell does not fire when the trigger is pulled, follow Commandment #1 and keep the firearm's muzzle pointed in a safe direction. Keep the muzzle down range with the action closed and wait at least 30 seconds (to ensure that the ammunition is not delayed in firing) before carefully opening the action, taking care to avoid exposure to the breech area and unloading the firearm and safely disposing of the ammunition.

Rule 7

Always Wear Eye and Ear Protection When Shooting

Exposure to shooting noise can permanently damage hearing. Flying debris, such as powder residue and ejected cartridge cases, can injure your eyes. Thus, it is only common sense to wear both eye protection (such as shooting glasses) and ear protection (such as a sound-muffling headset) whenever shooting. Also, wear eye protection when cleaning or disassembling your gun to ensure that cleaning solvent and tensioned parts (such as springs) do not come into contact with your eyes.

Rule 8

Be Sure the Barrel Is Clear of Obstructions Before Shooting. Discharging a firearm with an obstruction in the barrel can result in personal injury, property damage and/or death. Before you load your firearm, check the chamber and magazine to ascertain that no ammunition is inside. Also, check the inside of the barrel (called the "bore") to ensure it is free of obstructions. Even a small amount of mud, snow or excess lubricating oil or grease in the bore can cause excessive pressures resulting in a bulged or burst barrel which can injure or kill the shooter and bystanders. İt is a good idea to make a habit of cleaning the bore and checking for obstructions with a cleaning rod just before each shooting session. İf the noise or recoil experienced upon firing seems low or weak, or something doesn't feel "right," cease firing immediately and check to make sure that there is no obstruction in the barrel. Placing an undersized Shell or cartridge into a gun (such as a 20 gauge shell in a shotgun chambered for 12 gauge ammunition) can result in the smaller round of ammunition failing into the barrel and acting as an obstruction. When a round is subsequently fired, the barrel may burst, causing injury to the shooter and bystanders. For reference, re-read Commandment #5.

Rule 9

Do Not Alter or Modify Your Gun and Have It Serviced Regularly. Firearms are complex mechanisms that are designed to function properly in their original condition. Any alterations or changes made to a firearm after its manufacture can make the gun unsafe and will void its warranty. Do not jeopardize your safety or the safety of others by altering the trigger, mechanical safety or other mechanisms of your firearm. You should have your firearm periodically checked for proper functioning and serviced by a qualified gunsmith.

Rule 10

Learn the Mechanical and Handling Characteristics of Your Firearm. Not all firearms operate the same way. The method of carrying, handling and operating firearms varies with the mechanical characteristics of each gun. Thus, you should never handle any firearm until you become familiar with the safe handling, loading, unloading and carrying procedures for that particular firearm, as well as the rules for safe gun handling in general.

LEAD WARNING!

Discharging firearms in poorly ventilated areas, cleaning firearms or handling ammunition may result in exposure to lead and other substances known to cause birth defects, reproductive harm and other serious injuries. Have adequate ventilation at all times when shooting. Wash hands thoroughly after exposure.

Basics of Safe Gun Handling

1-	Always keep the muzzle pointed in a safe direction.

2-	Firearms should be unloaded when not actually in use.

3-	Don't totally rely on your gun's safety.

4-	Be sure of your target and what is beyond it.

5-	Use the correct ammunition for your firearm.

6-	If your gun fails to fire when the trigger is pulled, handle with care.

7-	Always wear eye and ear protection when shooting and cleaning.

8-	Be sure the barrel is clear of obstructions before shooting.

9-	Don't alter or modify your firearm and have your firearm(s) serviced regularly.

10-	Learn the mechanics and handling characteristics of the firearm you are using.

Safe gun handling depends on you! A safe shooter is a knowledgeable shooter.

WARNING! STOP SHOOTİNG IMMEDIATELY AND UNLOAD YOUR FİREARM if your firearm develops a mechanical malfunction; binding or stoppage, spitting powder/gas, a cartridge primer is punctured, a cartridge case is bulged or ruptured, or the sound on firing does not sound quite right. Do NOT try one more shot, but unload your firearm and take it and the ammunition to a qualified gunsmith. Do not assume that the firearm is empty merely because you checked the chamber. You must also check the bore for any obstruction.

WARNING! Always ensure that the ammunition you are using is clean, undamaged and of the proper caliber before loading your pistol. The manufacturer and distributor of this pistol disclaim any liability for the use of remanufactured, reloaded or hand-loaded ammunition. Protect your investment in this fine firearm by using only factory fresh, quality ammunition. The use of improper caliber or low-quality ammunition could result in injury, death and/or property damage upon firing. Remember, firearms safety depends on you! Follow all warnings provided by the manufacturer of the ammunition you are using.

LIABILITY

Our Products are classified as FIREARMS and are sold by AKDAŞ with the understanding that we will not be liable in any way for firearm malfunction, physical injury or property damage resulting in whole or in part from:

- Criminal or negligent shootings

- Improper, careless or malicious use

- Unauthorized modifications or replacement of parts

- Defective, inappropriate, hand-loaded or reloaded ammunition

- Corrosion/Maintenance neglect

- Other effects beyond our direct and immediate control

This limitation applies regardless of whether liability is based on contract, negligence or strict liability (including any failure to warn). AKDAŞ SİLAH A.Ş. In no event shall we be liable for incidental or consequential damages such as loss of use of property, business loss, loss of profits or profits.

Keep this manual next to your firearm. Make sure you understand the warnings, operating instructions and safety procedures. When loaning or selling a firearm, make sure this manual is included with the gun. You can request a copy of this manual from AKDAŞ SİLAH A.Ş.

THE SAFETY OF FIREARMS IS ENTIRELY THE RESPONSIBILITY OF THE SHOOTER. ALWAYS TREAT ALL FIREARMS AS LOADED!

IMPORTANT! READ THE SECTION AND WARNINGS IN THIS BOOKLET BEFORE USING THE FIREARM.

GENERAL INFORMATION

Congratulations on the purchase of AK-37UBFL, AK-40UBFL.

With proper care and use, it will give you long and reliable service. AK-37UBFL, AK-40UBFL is a launcher that can use 37/38mm and 40mm gas ammunition.

It has been produced to meet the needs of new generation infantry rifles and other infantry rifles with under-barrel Picatinny rail feature. The best features of the existing launchers are collected in a single weapon and a completely unique, ergonomic and user-friendly launcher is designed.

BASIC FEATURES

- Unique design for left and right use (safety latch and double side loading)

- Barrel direction lock that prevents switching to the left after the barrel is opened to the right and to the right after the barrel is opened to the left

- Possibility to use as a launcher rifle with telescopic stock in addition to the under-barrel use of the infantry rifle

- Bi-directional indication that the weapon is loaded

- High abrasion resistant, long-lasting, medium carbon steel barrel, hard chrome plated

- Ability to mount additional accessories on the top and sides thanks to the Picatinny rail system

- Lightweight and ergonomic for ease of use and transportation

- Double actiontrigger

foto

| AK-37UBFL |  |
| --- | --- |
| Caliber | 37 mm |
| Barrel Length | 190,5 mm (7,5") |
| Total Length (Stand Alone) (Stock Opened) | 623 mm (24,5") |
| Total Length (Stand Alone) (Stock Closed) | 547 mm (21,5") |
| Total Length (without stock for under barrel) | 273 mm (10,5") |
| Weapon Weight (Stand Alone-Unloaded-With Stock) | 2700 gr |
| Weapon Weight (Under Barrel Mounted-Unloaded-Without Stock) | 1700 gr |
| Sight System | 100-400 m (At 100 m intervals) |
| Fire Mode | Single Shoot |
| Fire Velocity | 5-6 Rounds / Minute |
| Muzzle Velocity | Minimum 76 m/s (±5m/s) |
| Effective Range | 50-150 metres |
| Trigger Pull Force | 25-35 Newton |
| Lock Force | 150-200 Newton |
| Life Span | 5000 Rounds |
| Shooting Pattern | 130 m'de 10 m Inside Radius Circle |
| Type of Ammo | 37/38mm Lighting and Gas cartridges |

AMMUNITION USED WITH AK-37UBFL MODEL LAUNCHERS: 37/38mm gas cartridge.

WARNING! It is forbidden to fire HE and similar ammunition with AK-37UBFL / AK-40UBFL, these models are designed to use only gas cartridges.

| AK-40UBFL |  |
| --- | --- |
| Caliber | 40 mm |
| Barrel Length | 190,5 mm (7,5") |
| Total Length (Stand Alone) (Stock Opened) | 623 mm (24,5") |
| Total Length (Stand Alone) (Stock Closed) | 547 mm (21,5") |
| Total Length (without stock for under barrel) | 273 mm (10,5") |
| Weapon Weight (Stand Alone-Unloaded-With Stock) | 2700 gr |
| Weapon Weight (Under Barrel Mounted-Unloaded-Without Stock) | 1700 gr |
| Sight System | 100-400 m (At 100 m intervals) |
| Fire Mode | Single Shoot |
| Fire Velocity | 5-6 Rounds / Minute |
| Muzzle Velocity | Minimum 76 m/s (±5m/s) |
| Effective Range | 50-150 metres |
| Trigger Pull Force | 25-35 Newton |
| Lock Force | 150-200 Newton |
| Life Span | 5000 Rounds |
| Shooting Pattern | 130 m'de 10 m Inside Radius Circle |
| Type of Ammo | 37/38mm Lighting and Gas cartridges |

AMMUNITION USED WITH AK-37UBFL MODEL LAUNCHERS: 37/38mm gas cartridge.

WARNING! It is forbidden to fire HE and similar ammunition with AK-37UBFL / AK-40UBFL, these models are designed to use only gas cartridges.

CARRYING BAG CONTENTS  BU ÇANTA MI KULLANILACAK

ÇANTA İÇERİĞİ

| 1 | AK-37UBFL / AK-40UBFL |  | 5 | Sling |
| --- | --- | --- | --- | --- |
| 2 | Stock Assembly |  | 6 | Ammunition Storage |
| 3 | Sight Assembly |  | 7 | Carrying Bag (Polyester Fabric) |
| 4 | Maintenance and Cleaning Kit |  | 8 | User Manual |

CLEANING KIT AND CONTENT

Resmi

CLEANING KIT CONTENT

| 1 | Cleaning Kit Bag | 7 | Barrel Cleaning Brush |
| --- | --- | --- | --- |
| 2 | 3-Piece Cleaning Slush | 8 | Cotton Brush |
| 3 | Cleaning Brush Thick (Steel Wire) | 9 | Oiler Tube |
| 4 | Cleaning Brush Fine (Steel Wire) | 10 | Slush cloth |
| 5 | Cleaning Brush Thin (Plastic Wire) | 11 | Flat Screwdriver |
| 6 | Cleaning Brush Thick (Plastic Wire) | 12 | Allen Key |

MOUNTING THE LAUNCHER ON AN ARMY RIFLE

The AK-37UBFL / AK-40UBFL launcher can be mounted on new generation infantry rifles and other infantry rifles with a Picatinny rail under the barrel. To mount the launcher, remove the retaining parts on the underside of the hand guard and hold it as shown in (Picture 17-a/b/c). Then, pull the infantry rifle backwards on the Picatinny rail until it reaches the end of the rail while pressing the screws on the Picatinny rail downwards. Upon releasing the screws, a distinct sound should be heard, indicating that the handguard connection rail is fully engaged with the mounts on the rifle. Tighten the Picatinny clamping butterflies on the right side of the launcher by turning them clockwise (refer to Figure 18). Once this process is completed, insert the Picatinny fixing pins manually (refer to Picture 19).

Resim 17-a			Resim 17-b		Resim 17-c

Resim 18			Resim 19

To attach the launcher to the telescopic or mobile stock for individual use, follow the same procedure as before, but do not fit the Picatinny fixing pins (see Figure 20-a/b/c/d/e). Use the allen spanner provided in the maintenance kit to insert and tighten the grip attachment screw, as shown in Figure 21. This completes the connection of the telescopic or mobile stock to the launcher body.

Resim 20-a        Resim 20-a      Resim 20-c

Resim 21

Resim 20-d		Resim 20-e

DISMOUNTING THE LAUNCHER FROM AN ARMY RIFLE

The installation phase requires a specific sequence of operations. First, remove the Picatinny fixing pins by hand. Then, loosen the screws of the Picatinny clamping jaws by turning them anti-clockwise. Next, press the screws connected to the handguard connection rail downwards and push the launcher forwards until the rifle is removed from the Picatinny rail.

TELESCOPIC STOCK AND SLING

The telescopic stock mounted on the launcher can be adjusted to the shooter's preference by pressing the latch (Figure 22) and gradually lengthening or shortening it (Figure 23-a/b).  Additionally, the telescopic stock has a stepped height adjustment feature that allows for the height of the cheek piece to be customized based on the user's facial structure and shooting distance (Picture 24).

Resim 22		Resim 23-a	         Resim 23-b	Resim 24

To assemble the sling belt that comes with the launcher, first place the belt ring with ball fitting structure onto the telescopic stock (see Picture 25). Then, attach the hook-type connection end of the sling belt to the belt ring (see Picture 26).

Resim 25

Resim 26

MOUNTING THE MECHANICAL SIGHT ON THE LAUNCHER AND PERFORMING A RESET

MOUNTING

- The mechanical sight can be mounted both on the Picatinny rails on the body of the launcher and on the side Picatinny rails of the rifle. Depending on the user's preference, it can be mounted on the right or left side.

- In both cases, the jaw and screw on the sight mount holder are removed and the mount holder is slid onto the side Picatinny rail (Figure 28).

- The holder screw is inserted from the upper side and the holder jaw is inserted from the lower side (Figure 29).

- Using the screwdriver included in the maintenance kit, tighten the screw to the end (Picture 30).

Resim 28-a	        Resim 28-b		Resim 29	       Resim 30

PERFORMING A RESET

1-After the sight is mounted on the rifle or launcher, the aiming adjustment is made on the target with a radius of 5 metres at a distance of 200 metres (Figure 31).

2-The sight is adjusted to 200 metres by pressing the distance adjustment lever.

3-Aim with the sight on the cross hair in the centre of the cloth target at 200 metres. 3 shots are fired.

4- Lateral correction is adjusted by loosening the lateral correction screw (Picture 32) to the centre hit point of the 3 shots (Picture 33).

5-Then the front sight screw is loosened (Picture 34) and the height of the front sight is adjusted according to the centre hit point (Picture 35) and the adjustment of the mechanical sight is completed by tightening the screw (Picture 34).

Resim 31	          Resim 32		     Resim 33

Resim 34		Resim 35

LOADING THE LAUNCHER

Before loading the AK-37UBFL / AK-40UBFL launcher, the safety switch is placed in the E (Safety) position (Picture 36). When the launcher is empty, the load indicator on the side surface of the lower receiver behind the barrel shows that the weapon is empty as shown in (Picture 37). In order to load the launcher, the barrel is opened forwards by pressing the barrel release latches on both sides of the upper receiver at the same time (Picture 38). The barrel is rotated to the right or left depending on whether the shooter is using the left or right side. The ammunition is placed into the barrel as shown in Figure 39. The barrel is grasped by the handguard and moved to the centre position and the barrel is pulled backwards until it is locked and the weapon is cocked (Picture 40). When the chamber of the launcher is loaded, the load indicator on both sides of the lower receiver will come out as shown in (Figure 41) and will warn the shooter that the weapon is loaded.

Resim 36		Resim 37		Resim 38

Resim 39		Resim 40		Resim 41

UNLOADING THE LAUNCHER

Before unloading the AK-37UBFL / AK-40UBFL launcher, the safety switch is set to E (Safety) position (Picture 36).  To unload the weapon, the barrel release latches on both sides of the upper receiver are pressed simultaneously (Picture 38). The barrel is opened forward, rotated to the right or left side and the empty cartridge case is removed from the barrel by grasping the bottom plate and pulling it backwards (Picture 42). If the firing is to be continued, reloading is performed. Then the barrel is grasped by the handguard and moved to the centre position and the weapon is switched off by pulling backwards until the barrel is locked (Picture 43).

Resim 42			          Resim 43

AK-37UBFL / AK-40UBFL launchers use full-round ammunition. One shot consists of ammunition, shell and bullet.

It can fire with 37/38mm and 40mm ammunition; fog, illumination, tear gas cartridges.

PREPARING THE LAUNCHER FOR FIRING

a.The inside of the barrel is checked to ensure that it is clean and dry.

b. The launcher is checked for cleanliness.

c.Check for faulty parts (parts that do not perform their function as they should).

d. The weapon is cocked with the gun unloaded and it is checked whether the trigger falls or not and whether the needle comes out after firing or not.

e.When the barrel release latch is pressed, it is checked that the barrel moves forward and opens to the right and left. When the barrel is pulled back, it is checked whether the weapon is cocked.

f. When the weapon is used individually or with an infantry rifle, it is checked whether the screws and fittings are fully functional. Loose connections may cause the weapon to break.

g.There should be no excessive oil in the trigger assembly, barrel and upper receiver of the weapon, and excess oil should be wiped off with a dry cloth.

AIMING AND FIRING

With the AK-37UBFL / AK-40UBFL launcher launcher, targets at a distance of up to 150 metres can be fired with suitable ammunition in individual use and mounted on an infantry rifle. After determining the target distance, the appropriate distance value is set by pressing the distance adjustment lever on the sight.  The target is aimed so that the white dots on the front sight and rear sight are at the bottom centre of the target (the sight is placed at the shooter's eye level) and the trigger is cocked by turning the safety switch to the A (fire) position (Figure 44).  There are divisions of 100, 200, 300 and 400 metres on the sight. As the distance increases, the angle of the barrel of the rifle to the horizontal plane increases, and as the distance decreases, this angle decreases.

ACTION IN CASE OF MISFIRE

- In the event of a jam (due to delayed firing or failure to fire), the following measures must be taken until the ammunition is removed from the weapon and the cause of the malfunction determined:

. Shout "MISFIRE", point the weapon towards the target, leaving no one in the direction of the barrel.

CAUTION! Before attempting to remove ammunition from the launcher, personnel in the area must be removed.

- Wait 1 minute before opening the barrel.

- Be careful during the unloading process.

- After the ammunition leaves the firing chamber, it is determined whether the ammunition or the firing system is faulty. It is checked whether there is a needle strike in the capsule. If the capsule is not crushed by the firing pin, the firing system is faulty. The ammunition can be reloaded after the cause of the firing error has been eliminated.

If the capsule is hit, wait for 1 minute and fire the weapon again. If the capsule is crushed and there is no fire, the shell is rotated 180 degrees and this process is repeated for the last time. If there is still no fire in the ammunition, that ammunition is left in a safe area away from other ammunition and is destroyed after firing.

ELIMINATING MISFIRE

| TYPE OF DETENTION | REASON FOR DETENTION | REPAIR AND OPERATION TO BE DONE |
| --- | --- | --- |
| Failure to Fire | - Safety - Empty firing chamber - Faulty ammunition - Excess water or oil on the firing pin - Wear or breakage of the firing pin - Covered trigger pulley or dirty firing pin - Weak or broken firing pin spring | - Take it to fire condition - Load the gun. - Refilled again - Cleaned - Replaced - Cleaned - Replaced |
| Cocking Failure | -Broken trigger pulley -Disregularity of the trigger cocking lever assembly | - Replaced - Replaced |
| Locking | -Broken lock carrier jamming or breakage, jamming of release latches | - Replaced |
| Ammunition Discharge Not Going to Bed | -Incorrect ammunition -Contaminated firing chamber | - Replaced - Cleaned |
| Hive Extraction Problem | -Exractor breakage, exractor jamming -Excessively contaminated ejector seat | - Replaced - Cleaned |
| Safety Pin Does Not Stay In Place | -Fracture of the safety release lever -Jamming in the safety housing | - Replaced - Adjustable |
| Front and Rear Do Not Stop in Selected Position | -Sight snap lock wear or breakage -Loosening or dislocation of the sight position screw | - Replaced - Tighten or replace the screw |

USER LEVEL CARE and CLEANING

- GENERAL INFORMATION

Maintenance is all measures taken to ensure that the launcher is always in working order. These measures include the following:

- a.	Cleaning

- b. Inspection of parts

- c. Repair of parts

- Oiling

2- EQUIPPING WITH CLEANING AND OILING MATERIALS

a. Cleaning Agents:

1. Barrel cleaning oil is used after firing. It also provides temporary protection against corrosion.

2. Hot soapy water or hot water is used when barrel cleaning oil is not available.

b. Oiling Agents:

2. Special protective oil is used at temperatures below -18 °C. (Addinol Weapon Oil W18)

3. SAE 10 engine oil or castor oil is to be used in combat conditions when the oils in 2.b (1) and (2) above are not available. As soon as possible, the Launcher lubricated with these oils should be cleaned and lubricated with its own oil.

c. Protective Oils:

1. Protective oil or grease against rust is used to protect the metal parts of the launcher during crating and storage.

3- LAUNCHER CLEANING

a. After firing, the ejector should be cleaned from capsule dust, powder sludge, metal crumbs and coal. Although the non-rusting composition of the firing capsule makes it easier to clean the launcher, this does not reduce the importance of cleaning. As a matter of fact, if the firing capsule residues are not cleaned, they will cause rusting by making a little moisture. The cleaning method described below removes all dirt and contamination other than metal crumbs. Fouling of the inside of the barrel with metal crumbs is rare and should only be cleaned by maintenance personnel. The ejector is disassembled after firing and cleaned as described.

  - 1. Inside the Barrel

b) A cotton slush cap is attached to the handle of the portable slush cap.

d) The slush cap is wetted with barrel cleaning oil, hot soapy water or just water.

f)	The inside of the barrel is then cleaned with a dry white cloth. Each time the cloth is removed, the cloth is examined. When there is no crumbs and residue on the cloth, the inside of the barrel is cleaned.

g)	The inside of the barrel is lightly lubricated and a thin layer of oil is applied to the inside of the barrel.

1. Firing Pin Retaining Cap

The firing pin retaining surface is cleaned and lightly lubricated with cloth and barrel cleaning oil using thick and thin wire cleaning brushes.

- Diğer Parçalar

- Dry cloth and bristle brush are used to clean other parts and exterior parts. All parts are wiped with a dry cloth. The metal parts are lightly lubricated with special protective oil.

- b. In cold weather (below 0°C): The ejector is kept in a damp-free environment and is not over lubricated. Dampness and excessive oil on moving parts will aggravate or completely stop the operation of these parts. The ejector must be disassembled and thoroughly cleaned with a dry cloth. If necessary, dry cleaning should be carried out to remove excess oil. Parts showing signs of wear are wiped with a slightly dampened cloth using special oil.

- If possible, maintain the Launcher in a closed and warm place when the gun is at room temperature.

- Oil the inner surfaces of the ejector, the working parts such as the mechanism, the springs and the inside of the trigger group. However, leave the outer surface as dry as possible.

- The launcher must be wrapped and covered when it is taken out of room temperature into cold air. Otherwise, a layer of moisture may form on the weapon due to sudden temperature change and cause rust.

- 4. When entering a warm environment (tent, etc.) from a cold environment, disassemble and dry the weapon.

- 5. Keep the weapon as dry as possible.

- 6. In extremely cold weather, perform a serviceability test every 30 minutes to prevent parts from freezing.

- 7. Never leave a hot and fired weapon on snow and ice.

- 8. When storing the weapon in the room, keeping the room temperature equal to the outside temperature can protect it against the adverse effects of sudden temperature changes.

9. If you place the weapon in a warm environment, dehumidify, clean and dry the weapon before returning it to the cold. Otherwise, ice may form on metal surfaces due to condensation.

c. In Hot Humid Weather:

- 1. Take normal care of the weapon.

- 2. Perform more frequent inspection, maintenance and oiling.

- 3. Continuously check the internal surfaces of the weapon for rust and corrosion.

- 4. Pay special attention to the rifling and sets. If possible, keep the inside of the barrel greased at all times.

- 5. Use maintenance oil more often.

- 6. If possible, keep the weapon closed.

7. Avoid contact with sea water and salt water. If the weapon is exposed to salt water, immediately carry out a complete disassembly and rinse all parts with potable water. Dry all parts and follow the normal maintenance procedure.

d. In Hot and Dry Weather:

- Sand inside the barrel and on moving parts must be cleaned every day. In sandy areas, the ejector must be kept dry to prevent sand accumulation. The muzzle and mechanism group should be protected against sand and dust storms by covering.  When the dusty and sandy conditions disappear and signs of rusting start to show, the ejector should be lubricated again.

- 2. Carry out weapon maintenance frequently.

- 3. Care should be taken in oiling as excessive oiling of the gun will cause sludge on the gun.

- 4. The inner surface of the weapon and the working parts should be lubricated, but the outer surface should be left dry.

- 5. Rust and corrosion are not common in this type of terrain, but check the weapon anyway.

- 6. If possible, weapons should be kept closed.

- 7. The inside of the barrel must be continuously blasted against dust and sand grains and kept dry.

- 8. Keep the weapon in a carrying case or transport crate to protect it from sand.

- 9. Clean the barrel and chamber daily.

- 10. Prevent sand from entering the barrel.

11. Thoroughly drying and lightly oiling the weapon prevents the liquid produced by sweating hands and cheeks from causing rust.

- Barrel Cleaning:

- - Place the weapon on a cleaning table or other flat surface with the muzzle pointing away from the target. Be careful not to bump the muzzle of the weapon against the table.

- - Always clean the barrel from the chamber to the muzzle, keeping the muzzle lower to prevent maintenance oil from entering the loading and firing mechanisms.

- - Thoroughly clean the slusher before starting barrel cleaning.

- - When cleaning the barrel with the slusher, always work from the firing chamber to the end of the barrel. After the slusher is removed from the end of the barrel, remove the cloth and unscrew the end of the slusher. Then pull the slusher backwards and repeat the process.

- - Firstly, attach a dry and clean cloth to the barrel and dry clean the barrel several times.

- Attach any cloth moistened with maintenance oil to the end of the barrel and oiling the inside of the barrel.

- Wait 15-20 minutes for the maintenance oil to remove the residues inside the barrel.

- Attach the bronze barrel brush to the barrel tip and moisten it with oil. The number of times the barrel is cleaned is calculated as 1 time for every 3 shots with the bronze barrel brush and this number of times is applied.

- Then continue cleaning with a cotton brush. Attach a dry and clean cloth to the barrel (a barrel cloth is included in the cleaning kit) and repeat the process until it comes out clean.

- To clean the barrel chamber, attach a steel-wire chamber brush (thin or thick, as appropriate) to the harbour end and brush the chamber.

- Remove dust and dirt from various parts of the weapon with the plastic wire brushes included in the cleaning kit

WARNING! When cleaning the inside of the barrel with a muzzle brush and muzzle cloth, never pull the muzzle from the end of the barrel towards the firing chamber!

WARNING! Never use force when disassembling, cleaning and assembling the weapon!

STORAGE

    - Preparation for Storage: Before your firearm is stored, it should be thoroughly cleaned and lightly oiled. A thin layer of high-quality firearm oil should be applied to all exposed surfaces. In humid conditions, ensure that cleaning cloths used are dry. After the metal parts are dried, they should not be touched with bare hands. For normal storage periods, metal parts are lightly oiled with protective oil, and for longer periods, they are oiled with grease. The firearm should never be wrapped with cloth or similar material, and no plug should be placed in the barrel, as these can cause the firearm to rust by absorbing moisture.

    - Cleaning After Long-Term Storage: A firearm taken out of storage after an extended period should be thoroughly cleaned and oiled with normal protective oil. If it is to be fired immediately, ensure that the oil used for storage (protective or grease) is completely removed, the inside of the barrel is dry, and there is no obstruction before firing.

BEFORE STORING, CHECK IF YOUR SEMI-AUTOMATIC PISTOL IS LOADED. STORE FIREARMS AND AMMUNITION SEPARATELY AND IN A SECURE PLACE INACCESSIBLE TO UNAUTHORIZED INDIVIDUALS. ALWAYS LOCK YOUR FIREARMS SAFELY WHEN NOT IN USE. CONSULT YOUR COUNTRY'S FIREARMS REGULATIONS FOR THE SAFE STORAGE OF FIREARMS AND AMMUNITION.

WARNING! Modifying or changing parts and/or internal safeties is dangerous and voids the warranty. This firearm is designed to function properly with original parts. It is your responsibility to ensure that all purchased parts are intended for this firearm, installed correctly, and both spare and original parts are not replaced or modified. Your firearm is a complex and delicate tool with many parts that need to be correctly associated with other parts for proper and safe operation. Assembling a firearm with incorrect or modified parts can lead to damage, danger, and malfunction, resulting in injury or death to yourself and others. Always consult a qualified gunsmith for any work on your firearm.

SPARE PARTS CATALOGUE

Main Parts and Assembly of the Weapon

Part Names