WARNINGS, SAFETY RULES

GENERAL INFORMATION

BASIC FEATURES

TECHNICAL SPECIFICATION

AK-40GLJ

AK-40GLX

LAYOUT OF THE GRENADE LAUNCHER İN THE CARRY BAG

CLEANING KIT AND ITS CONTENTS

ATTACHING THE GRENADE LAUNCHER TO THE INFANTRY RIFLE

DISMANTLING OF THE GRENADE LAUNCHER FROM THE INFANTRY RIFLE

ATTACHING THE MECHANICAL SIGHT TO THE GRENADE LAUNCHER AND ZEROING

TELESCOPIC STOCK AND SLING

LOADING THE GRENADE LAUNCHER

UNLOADING THE GRENADE LAUNCHER

PREPARING THE LAUNCHER TO FIRE

AIMING AND SHOOTING WITH THE GRENADE LAUNCHER

SHOOTING POSITİONS

ACTION IN CASE OF MALFUNCTIONS

REMOVAL OF MALFUNCTİONS

USER LEVEL MAINTENANCE AND CLEANING

ADVANCED MAINTENANCE-REPAIR WARNINGS

STORAGE

USER DAILY and WEEKLY MAINTENANCE FORM

SPARE PARTS CATALOGUE

Exploded View

Part List

IMPORTANT SAFETY MESSAGE

Children are attracted to and can operate firearms which can cause severe injuries or death. Prevent child access by always keeping gun locked, unloaded and separated from ammunition when not in use. If you keep a loaded firearm where a child obtains and improperly uses it, you may be fined or sent to prison.

Firearm Safety Depends on You

A gun is only as safe as the person operating it. You can never be overly careful when handling a firearm.

Carelessness is often the cause of shooting accidents, such as failing to keep the muzzle pointed in a safe direction, not being sure of your target and what is behind it, failing to properly engage the safety, leaving ammunition in the chamber or using improper loads. Since a bullet can never be called back once fired, such errors in gun handling can result in the loss of life, severe injury or property damage thus, it is crucial for your safety and the safety of those around you that you learn the principles of safe gun handling and storage before you begin to use your new firearm. Be a safe shooter - please read this instruction book thoroughly even if this is not your first firearm purchase as not all firearms are the same. The first step in being a safe shooter is to learn the rules for the safe operation and handling of firearms. There is nothing more important in gun handling than safety.

THE TEN RULES OF FİREARM SAFETY

The Ten Rules of Firearm Safety must be etched into your memory before you begin to handle firearms. These rules are intended to be followed by all persons handling firearms in the field, on the range or at home. Please read, review and understand these rules before you begin to use or even take your new firearm out of its box. Remember, firearms safety depends on you! Memorizing these safety rules will help prevent gun accidents. Please study these safety rules before handling your firearm.

Rule 1

Always Keep the Muzzle Pointed in a Safe Direction

This is the most basic and most important safety rule. A safe direction is one in which an accidental discharge will not cause injury to yourself, to others or property damage. This is particularly important when loading or unloading your firearm. Never point your gun at anything you do not intend to shoot. Treat every gun as if it were loaded at all times.

Rule 2

Firearms Should Be Unloaded When Not Actually in Use

Firearms should only be loaded when you are in the field or on the target range or shooting area, ready to shoot. When not in use, firearms and ammunition should be secured in a safe place, separate from each other, Remember to unload your firearm completely so that there is no ammunition in the chamber or magazine. Before handling this or any firearm, or handing it to someone else, visually check the chamber. Never assume a gun is unloaded even İf you were the last person to use it. Never cross a fence, climb a tree, wade through a stream or perform any awkward movement with a loaded gun. When in doubt, unload your gun! Never pull or push a loaded firearm toward yourself or another person. Never carry a loaded gun in a scabbard, a holster not being worn or a gun case - common sense prevails in gun safety!

Rule 3

Don't Completely Rely on Your Gun's Safety

Treat every gun as though it could fire at any time, even if you are not applying pressure to the trigger. The "safety" on a firearm is a mechanical device which, like any such device, can become inoperable at the worst possible time and fail to function. By mistake, you may think the safety is "ON" when it actually is not. Or you may think your gun is unloaded when there is actually a round of ammunition in it. The safety serves as a supplement to proper gun handling but cannot serve as a substitute for common sense. Never handle a gun carelessly and assume that the gun won't fire just because "the safety is on." Never touch the firearm's trigger until you are ready to shoot. Keep your fingers away from the trigger when loading or unloading. Never pull the trigger when the safety is engaged or when the safety is positioned between the "SAFE" and "FİRE" positions.

Alcohol, Drugs and Guns Don't Mix. Make No Mistake About It!

Never handle firearms after consuming alcohol or taking drugs that can affect your judgment. Shoot sober! Alcohol, certain kinds of drugs and firearms don't mix. Safe firearms handling requires alertness and concentration on one's actions. You cannot handle a firearm safely after consuming alcohol. Never consume anything that can impair your judgment or physical coordination when handling a firearm.

Rule 4

Be Sure of Your Target - And What Is Beyond It!

Once fired, a bullet (or shot charge) can never be called back; so before you shoot, know where the bullet is going and what it will strike. Be certain your shot will not injure someone or strike something beyond the target. Never fire in the direction of noise, a movement or at any object you cannot positively identify. Be aware that a .22 Short bullet can travel over 1 % miles. A centerfire cartridge, such as the .30-06, can send its bullet over 3 miles. Shotgun pellets can travel 500 yards and a shotgun slug has a range of over a half mile. Make sure your shot has a safe backstop such as a hillside. Keep in mind how far the bullet will travel if it misses your intended target. You are responsible for your actions and judgment.

Rule 5

Use the Correct Ammunition

Every firearm is designed to use a certain caliber or gauge of ammunition. İt is important that you use the correct ammunition for your firearm. Information on the correct ammunition to use with your firearm appears in the firearm's instruction manual and the manufacturer's markings on the firearm itself. Use of the wrong ammunition, improperly reloaded ammunition or corroded ammunition can result in the destruction of the firearm, serious personal injury and/or death. Form the habit of examining every round of ammunition before you put it into your gun to ensure it is of the proper gauge or caliber and that it is in good condition.

Rule 6

İf Your Gun Fails to Fire When the Trigger Is Pulled, Handle With Care. İf a cartridge or shell does not fire when the trigger is pulled, follow Commandment #1 and keep the firearm's muzzle pointed in a safe direction. Keep the muzzle down range with the action closed and wait at least 30 seconds (to ensure that the ammunition is not delayed in firing) before carefully opening the action, taking care to avoid exposure to the breech area and unloading the firearm and safely disposing of the ammunition.

Rule 7

Always Wear Eye and Ear Protection When Shooting

Exposure to shooting noise can permanently damage hearing. Flying debris, such as powder residue and ejected cartridge cases, can injure your eyes. Thus, it is only common sense to wear both eye protection (such as shooting glasses) and ear protection (such as a sound-muffling headset) whenever shooting. Also, wear eye protection when cleaning or disassembling your gun to ensure that cleaning solvent and tensioned parts (such as springs) do not come into contact with your eyes.

Rule 8

Be Sure the Barrel Is Clear of Obstructions Before Shooting. Discharging a firearm with an obstruction in the barrel can result in personal injury, property damage and/or death. Before you load your firearm, check the chamber and magazine to ascertain that no ammunition is inside. Also, check the inside of the barrel (called the "bore") to ensure it is free of obstructions. Even a small amount of mud, snow or excess lubricating oil or grease in the bore can cause excessive pressures resulting in a bulged or burst barrel which can injure or kill the shooter and bystanders. İt is a good idea to make a habit of cleaning the bore and checking for obstructions with a cleaning rod just before each shooting session. İf the noise or recoil experienced upon firing seems low or weak, or something doesn't feel "right," cease firing immediately and check to make sure that there is no obstruction in the barrel. Placing an undersized Shell or cartridge into a gun (such as a 20 gauge shell in a shotgun chambered for 12 gauge ammunition) can result in the smaller round of ammunition failing into the barrel and acting as an obstruction. When a round is subsequently fired, the barrel may burst, causing injury to the shooter and bystanders. For reference, re-read Commandment #5.

Rule 9

Do Not Alter or Modify Your Gun and Have It Serviced Regularly. Firearms are complex mechanisms that are designed to function properly in their original condition. Any alterations or changes made to a firearm after its manufacture can make the gun unsafe and will void its warranty. Do not jeopardize your safety or the safety of others by altering the trigger, mechanical safety or other mechanisms of your firearm. You should have your firearm periodically checked for proper functioning and serviced by a qualified gunsmith.

Rule 10

Learn the Mechanical and Handling Characteristics of Your Firearm. Not all firearms operate the same way. The method of carrying, handling and operating firearms varies with the mechanical characteristics of each gun. Thus, you should never handle any firearm until you become familiar with the safe handling, loading, unloading and carrying procedures for that particular firearm, as well as the rules for safe gun handling in general.

LEAD WARNING!

Discharging firearms in poorly ventilated areas, cleaning firearms or handling ammunition may result in exposure to lead and other substances known to cause birth defects, reproductive harm and other serious injuries. Have adequate ventilation at all times when shooting. Wash hands thoroughly after exposure.

Basics of Safe Gun Handling

1-	Always keep the muzzle pointed in a safe direction.

2-	Firearms should be unloaded when not actually in use.

3-	Don't totally rely on your gun's safety.

4-	Be sure of your target and what is beyond it.

5-	Use the correct ammunition for your firearm.

6-	If your gun fails to fire when the trigger is pulled, handle with care.

7-	Always wear eye and ear protection when shooting and cleaning.

8-	Be sure the barrel is clear of obstructions before shooting.

9-	Don't alter or modify your firearm and have your firearm(s) serviced regularly.

10-	Learn the mechanics and handling characteristics of the firearm you are using.

Safe gun handling depends on you! A safe shooter is a knowledgeable shooter.

WARNING! STOP SHOOTİNG IMMEDIATELY AND UNLOAD YOUR FİREARM if your firearm develops a mechanical malfunction; binding or stoppage, spitting powder/gas, a cartridge primer is punctured, a cartridge case is bulged or ruptured, or the sound on firing does not sound quite right. Do NOT try one more shot, but unload your firearm and take it and the ammunition to a qualified gunsmith. Do not assume that the firearm is empty merely because you checked the chamber. You must also check the bore for any obstruction.

WARNING! Always ensure that the ammunition you are using is clean, undamaged and of the proper caliber before loading your pistol. The manufacturer and distributor of this pistol disclaim any liability for the use of remanufactured, reloaded or hand-loaded ammunition. Protect your investment in this fine firearm by using only factory fresh, quality ammunition. The use of improper caliber or low-quality ammunition could result in injury, death and/or property damage upon firing. Remember, firearms safety depends on you! Follow all warnings provided by the manufacturer of the ammunition you are using.

LIABILITY

Our Products are classified as FIREARMS and are sold by AKDAŞ with the understanding that we will not be liable in any way for firearm malfunction, physical injury or property damage resulting in whole or in part from:

- Criminal or negligent shootings

- Improper, careless or malicious use

- Unauthorized modifications or replacement of parts

- Defective, inappropriate, hand-loaded or reloaded ammunition

- Corrosion/Maintenance neglect

- Other effects beyond our direct and immediate control

This limitation applies regardless of whether liability is based on contract, negligence or strict liability (including any failure to warn). AKDAŞ SİLAH A.Ş. In no event shall we be liable for incidental or consequential damages such as loss of use of property, business loss, loss of profits or profits.

Keep this manual next to your firearm. Make sure you understand the warnings, operating instructions and safety procedures. When loaning or selling a firearm, make sure this manual is included with the gun. You can request a copy of this manual from AKDAŞ SİLAH A.Ş.

THE SAFETY OF FIREARMS IS ENTIRELY THE RESPONSIBILITY OF THE SHOOTER. ALWAYS TREAT ALL FIREARMS AS LOADED!

IMPORTANT! READ THE SECTION AND WARNINGS IN THIS BOOKLET BEFORE USING THE FIREARM.

GENEL BİLGİLER

Congratulations on the purchase of AK-40GLJ, AK-40GLX.

It will give you long and reliable service with proper maintenance and use. AK-40GLJ/AK-40GLX is a grenade launcher weapon that can use 40mm low velocity ammunition.

This grenade launcher has been designed and manufactured to be used with the new generation comprising a Picatinny rail under the barrel. This 40mm grenade launcher is unique gathering all the superior features of various weapons together. The best features of the existing grenade launchers are collected in a single gun and a completely unique 40 mm grenade launcher has been designed. AK-40GLJ/AK-40GLX has superior features than existing grenade launchers with its range, strength, ease of installation and disassembly, original design sighting system, and ease of use.

BASIC SPECIFICATION

-	Light and ergonomic design for ease of use and transport

-	Unique design for right- and left-hand use (safety latch and double-sided loading)

-	In addition to the utilization by attaching to an infantry rifle, capability of using as a standalone launcher gun comprising a telescopic stock

-	Double-sided indicator that shows whether the launcher is loaded

-	Mechanical sight system that allows night shooting, phosphorescent and can change the direction of mounting

-	High wear resistant, hard chrome coated barrel with mid carbon alloy that provides long barrel life span

-	Picatinny rails on both sides that allow additional equipment

-	Thanks to the direction lock, the barrel is prevented from switching to the right after opening to the left, switching to the left after opening to the right

TECHNICAL SPECIFICATIONS

foto

| AK-40GLJ |  |
| --- | --- |
| Caliber | 40 mm |
| Barrel Length | 190,5 mm (7,5") |
| Total Length Standalone (Telescopic Stock Open) | 620 mm (24,4") |
| Total Length Standalone (Telescopic Stock Close) | 545 mm (21,4") |
| Total Length Attached to Rifle | 270 mm (10,6") |
| Weight (Standalone-Empty) | 2450 gr |
| Weight (Attached to Rifle-Empty) | Max.1500 gr |
| Sight System | 100-400 m (100 m intervals) |
| Shooting Mode | Single Shot |
| Shooting Velocity | 5-6 rounds / minute |
| Muzzle Velocity | Minimum 75 (±5) m/s |
| Effective Range | 400 m |
| Trigger Pull Force | 3-6,5 kg (45 Newton) |
| Coking Force | 6-9 kg (65-90 Newton) |
| Barrel Life Span | 5000 rounds |
| Shooting Pattern | Into 5 m diameter at 200 m |
| Type of Ammunition | 40x46 mm new and old generation low velocity (LV) ammunition of any type |

| AK-40GLX |  |
| --- | --- |
| Caliber | 40 mm |
| Barrel Length | 254 mm (10") |
| Total Length Standalone (Telescopic Stock Open) | 685 mm (27") |
| Total Length Standalone (Telescopic Stock Close) | 610 mm (24") |
| Total Length Attached to Rifle | 337 mm (13.25") |
| Weight (Standalone-Empty) | 2700 gr |
| Weight (Attached to Rifle-Empty) | Max.1500 gr |
| Sight System | 100-400 m (100 m intervals) |
| Shooting Mode | Single Shot |
| Shooting Velocity | 5-6 rounds / minute |
| Muzzle Velocity | Minimum 75 (±5) m/s |
| Effective Range | 400 m |
| Trigger Pull Force | 3-6,5 kg (45 Newton) |
| Coking Force | 6-9 kg (65-90 Newton) |
| Barrel Life Span | 5000 rounds |
| Shooting Pattern | Into 5 m diameter at 200 m |
| Type of Ammunition | 40x46 mm new and old generation low velocity (LV) ammunition of any type |

LAYOUT OF THE GRENADE LAUNCHER İN THE CARRY BAG

PACK INCLUDES

| 1 | AK-40GLJ/AK-40GLX Grenade Launcher |  | 5 | Sling |
| --- | --- | --- | --- | --- |
| 2 | Stock Assembly |  | 6 | Ammunition Storage |
| 3 | Sight Assembly |  | 7 | Carrying Bag (Polyester Fabric) |
| 4 | Maintenance and Cleaning Kit |  | 8 | User Manual |

Resmi

CLEANING KIT AND COMPONENTS

| 1 | Cleaning Kit Bag | 7 | Barrel Cleaning Brush |
| --- | --- | --- | --- |
| 2 | 3-Piece Cleaning Slush | 8 | Cotton Brush |
| 3 | Cleaning Brush Thick (Steel Wire) | 9 | Oiler Tube |
| 4 | Cleaning Brush Fine (Steel Wire) | 10 | Slush cloth |
| 5 | Cleaning Brush Thin (Plastic Wire) | 11 | Flat Screwdriver |
| 6 | Cleaning Brush Thick (Plastic Wire) | 12 | Allen Key |

MOUNTING THE LAUNCHER ON AN ARMY RIFLE

The AK-40GLX / AK-40GLJ launcher can be mounted on new generation infantry rifles and other infantry rifles with a Picatinny rail under the barrel. To mount the launcher, remove the retaining parts on the underside of the hand guard and hold it as shown in (Picture 17-a/b/c). Then, pull the infantry rifle backwards on the Picatinny rail until it reaches the end of the rail while pressing the screws on the Picatinny rail downwards. Upon releasing the screws, a distinct sound should be heard, indicating that the handguard connection rail is fully engaged with the mounts on the rifle. Tighten the Picatinny clamping butterflies on the right side of the launcher by turning them clockwise (refer to Figure 18). Once this process is completed, insert the Picatinny fixing pins manually (refer to Picture 19).

Resim 17-a			Resim 17-b		Resim 17-c

Resim 18			Resim 19

To attach the launcher to the telescopic or mobile stock for individual use, follow the same procedure as before, but do not fit the Picatinny fixing pins (see Figure 20-a/b/c/d/e). Use the allen spanner provided in the maintenance kit to insert and tighten the grip attachment screw, as shown in Figure 21. This completes the connection of the telescopic or mobile stock to the launcher body.

Resim 20-a        Resim 20-a      Resim 20-c

Resim 21

Resim 20-d		Resim 20-e

DISMOUNTING THE LAUNCHER FROM AN ARMY RIFLE

The installation phase requires a specific sequence of operations. First, remove the Picatinny fixing pins by hand. Then, loosen the screws of the Picatinny clamping jaws by turning them anti-clockwise. Next, press the screws connected to the handguard connection rail downwards and push the launcher forwards until the rifle is removed from the Picatinny rail.

TELESCOPIC STOCK AND SLING

The telescopic stock mounted on the launcher can be adjusted to the shooter's preference by pressing the latch (Figure 22) and gradually lengthening or shortening it (Figure 23-a/b).  Additionally, the telescopic stock has a stepped height adjustment feature that allows for the height of the cheek piece to be customized based on the user's facial structure and shooting distance (Picture 24).

Resim 22		Resim 23-a	         Resim 23-b	Resim 24

To assemble the sling belt that comes with the launcher, first place the belt ring with ball fitting structure onto the telescopic stock (see Picture 25). Then, attach the hook-type connection end of the sling belt to the belt ring (see Picture 26).

Resim 25

Resim 26

MOUNTING THE MECHANICAL SIGHT ON THE LAUNCHER AND PERFORMING A RESET

MOUNTING

- The mechanical sight can be mounted both on the Picatinny rails on the body of the launcher and on the side Picatinny rails of the rifle. Depending on the user's preference, it can be mounted on the right or left side.

- In both cases, the jaw and screw on the sight mount holder are removed and the mount holder is slid onto the side Picatinny rail (Figure 28).

- The holder screw is inserted from the upper side and the holder jaw is inserted from the lower side (Figure 29).

- Using the screwdriver included in the maintenance kit, tighten the screw to the end (Picture 30).

Resim 28-a	        Resim 28-b		Resim 29	       Resim 30

PERFORMING A RESET

1-After the sight is mounted on the rifle or launcher, the aiming adjustment is made on the target with a radius of 5 metres at a distance of 400 metres (Figure 31).

2-The sight is adjusted to 400 metres by pressing the distance adjustment lever.

3-Aim with the sight on the cross hair in the centre of the cloth target at 400 metres. 3 shots are fired.

4- Lateral correction is adjusted by loosening the lateral correction screw (Picture 32) to the centre hit point of the 3 shots (Picture 33).

5-Then the front sight screw is loosened (Picture 34) and the height of the front sight is adjusted according to the centre hit point (Picture 35) and the adjustment of the mechanical sight is completed by tightening the screw (Picture 34).

Resim 31	          Resim 32		     Resim 33

Resim 34		Resim 35

| ELEVATION CORRECTION (WITH 1 LINE) |  |
| --- | --- |
| 100 m | ± 5 m |
| 200 m | ± 7 m |
| 300 m | ± 10 m |
| 400 m | ± 13 m |

| LATERAL CORRECTION (WITH 1 LINE) |  |
| --- | --- |
| 100 m | ± 5 m |
| 200 m | ± 12 m |
| 300 m | ± 19 m |
| 400 m | ± 26 m |

LOADING THE LAUNCHER

Before loading the AK-40GLX / AK-40GLJ launcher, the safety switch is placed in the E (Safety) position (Picture 36). When the launcher is empty, the load indicator on the side surface of the lower receiver behind the barrel shows that the weapon is empty as shown in (Picture 37). In order to load the launcher, the barrel is opened forwards by pressing the barrel release latches on both sides of the upper receiver at the same time (Picture 38). The barrel is rotated to the right or left depending on whether the shooter is using the left or right side. The ammunition is placed into the barrel as shown in Figure 39. The barrel is grasped by the handguard and moved to the centre position and the barrel is pulled backwards until it is locked and the weapon is cocked (Picture 40). When the chamber of the launcher is loaded, the load indicator on both sides of the lower receiver will come out as shown in (Figure 41) and will warn the shooter that the weapon is loaded.

Resim 36		Resim 37		Resim 38

Resim 39		Resim 40		Resim 41

UNLOADING THE LAUNCHER

Before unloading the AK-40GLX / AK-40GLJ launcher, the safety switch is set to E (Safety) position (Picture 36).  To unload the weapon, the barrel release latches on both sides of the upper receiver are pressed simultaneously (Picture 38). The barrel is opened forward, rotated to the right or left side and the empty cartridge case is removed from the barrel by grasping the bottom plate and pulling it backwards (Picture 42). If the firing is to be continued, reloading is performed. Then the barrel is grasped by the handguard and moved to the centre position and the weapon is switched off by pulling backwards until the barrel is locked (Picture 43).

Resim 42			          Resim 43

AK-40GLJ / AK-40GLX grenade launcher uses a full round of ammunition. One round consists of ammunition, shell and bullet.

From 40mm ammunition; It can fire with low-speed destruction, thermobaric, air-exploding, fog, illumination, tear gas, training and tracer training rounds.

PREPARING THE LAUNCHER FOR FIRING

a.The inside of the barrel is checked to ensure that it is clean and dry.

b. The launcher is checked for cleanliness.

c.Check for faulty parts (parts that do not perform their function as they should).

d. The weapon is cocked with the gun unloaded and it is checked whether the trigger falls or not and whether the needle comes out after firing or not.

e.When the barrel release latch is pressed, it is checked that the barrel moves forward and opens to the right and left. When the barrel is pulled back, it is checked whether the weapon is cocked.

f. When the weapon is used individually or with an infantry rifle, it is checked whether the screws and fittings are fully functional. Loose connections may cause the weapon to break.

g.There should be no excessive oil in the trigger assembly, barrel and upper receiver of the weapon, and excess oil should be wiped off with a dry cloth.

NİŞAN ALMA ve ATIŞ

The AK-40GLJ / AK-40GLX grenade launcher can be used individually and mounted on an infantry rifle to fire at targets up to 400 metres away with suitable ammunition. After determining the target distance, the appropriate distance value is set by pressing the distance adjustment lever on the sight.  The target is aimed so that the white dots on the front sight and rear sight are at the bottom centre of the target (the sight is placed at the shooter's eye level) and the trigger is cocked by turning the safety switch to the A (fire) position (Figure 44).  There are 100, 200, 300 and 400 metre divisions on the sight. As the distance increases, the angle that the barrel of the weapon makes to the horizontal plane increases, and as the distance decreases, this angle decreases. (Figures 45, 46, 47 and 48) show the firing angles according to the distance.

Resim 45					Resim 46

Resim 47						Resim 48

ATIŞ POZİSYONLARI

Ayakta Atış Pozisyonu

Çökerek Atış Pozisyonu

Yatarak Atış Pozisyonu

ACTION IN CASE OF MISFIRE

- In the event of a jam (due to delayed firing or failure to fire), the following measures must be taken until the ammunition is removed from the weapon and the cause of the malfunction determined:

. Shout "MISFIRE", point the weapon towards the target, leaving no one in the direction of the barrel.

CAUTION! Before attempting to remove ammunition from the launcher, personnel in the area must be removed.

- Wait 1 minute before opening the barrel.

- Be careful during the unloading process.

- After the ammunition leaves the firing chamber, it is determined whether the ammunition or the firing system is faulty. It is checked whether there is a needle strike in the capsule. If the capsule is not crushed by the firing pin, the firing system is faulty. The ammunition can be reloaded after the cause of the firing error has been eliminated.

If the capsule is hit, wait for 1 minute and fire the weapon again. If the capsule is crushed and there is no fire, the shell is rotated 180 degrees and this process is repeated for the last time. If there is still no fire in the ammunition, that ammunition is left in a safe area away from other ammunition and is destroyed after firing.

ELIMINATING MISFIRE

| TYPE OF DETENTION | REASON FOR DETENTION | REPAIR AND OPERATION TO BE DONE |
| --- | --- | --- |
| Failure to Fire | - Safety - Empty firing chamber - Faulty ammunition - Excess water or oil on the firing pin - Wear or breakage of the firing pin - Covered trigger pulley or dirty firing pin - Weak or broken firing pin spring | - Take it to fire condition - Load the gun. - Refilled again - Cleaned - Replaced - Cleaned - Replaced |
| Cocking Failure | -Broken trigger pulley -Disregularity of the trigger cocking lever assembly | - Replaced - Replaced |
| Locking | -Broken lock carrier jamming or breakage, jamming of release latches | - Replaced |
| Ammunition Discharge Not Going to Bed | -Incorrect ammunition -Contaminated firing chamber | - Replaced - Cleaned |
| Hive Extraction Problem | -Exractor breakage, exractor jamming -Excessively contaminated ejector seat | - Replaced - Cleaned |
| Safety Pin Does Not Stay In Place | -Fracture of the safety release lever -Jamming in the safety housing | - Replaced - Adjustable |
| Front and Rear Do Not Stop in Selected Position | -Sight snap lock wear or breakage -Loosening or dislocation of the sight position screw | - Replaced - Tighten or replace the screw |

USER-LEVEL MAINTENANCE AND CLEANING

1-	GENERAL INFORMATION

Maintenance is all the measures taken to keep the grenade launcher always operational. These measures include the following considerations:

a.	Cleaning

b.	Inspection of parts

c.	Repair of parts

d.	Lubrication

2-	EQUIPMENT WITH CLEANİNG AND LUBRICATING AGENTS

a.	Cleaning Materials:

- Oil for cleaning interior of the barrel is used after shooting. İt also provides temporary protection against rust.

- Hot soapy water or just hot water are used when oil for cleaning inside the barrel is not available.

b.	Lubricants:

- Common lubricating oil is used to lubricate the grenade launcher for temperatures above 18°C.

- Special lubricating oil is used to lubricate the grenade launcher for temperatures below -18°C (Addinol weapon Oil W18).

- SAE 10 engine oil or castor oil can be used at war conditions when above-mentioned oils are not available. At the earliest time, grenade launchers lubricated with these oils must be cleaned and lubricated with proper oils.

c.	Protective Oils:

1.	When storing grenade launcher in crates, protective oils or grease must be used to protect metal parts.

3-	CLEANİNG THE FLARE LAUNCHER

After shooting all the residue arising from primer, powder, and any other stuff must be cleaned. Even though the rust-free structure of the primer makes the cleaning easier, cleaning must never be neglected. Indeed, rusting may occur due to the humidity arising from the primer in the long run if the residue from primers is not cleaned. The below mentioned cleaning method will remove any dirt except for the metal chips. Barrels rarely get dirty of metal chips and this is cleaned by professionals only. After firing, the grenade launcher is disassembled and cleaned as described;

1.	Inside the barrel:

(a)	Cotton brush is attached to the cleaning rod.

(b)	Cotton brush is wetted with the lubricant for barrel bore, hot soapy water or just water.

(c)	Cotton brush is inserted in and moved forward-backward a couple of times inside the barrel.

(d)	Next, wire brush is attached to the cleaning rod and again, moved forward-backward a couple of times inside barrel.

(e)	Using a dry cloth, barrel bore is cleaned. Cleaning cloth is checked every time it goes out. After making sure that no residue is left on the cloth, it means that of bore of the barrel is cleaned.

(f)	Finally, interior of the barrel is lubricated lightly inside to form a layer of oil.

2.	Firing Pin Housing:

Firing pin housing is cleaned using cloths, thin and thick wire cleaning brushes and lightly lubricated with bore cleaning oil.

3.	Other Parts:

a.	Dry cloth and a clay brush are used to clean other parts and exteriors. Ali parts are wiped with dry cloth. Metal parts are lightly lubricated with special protective oil.

b.	İn Cold weather (below 0°C): The Grenade Launcher is stored in a non-humid environment and is not over-oiled. Moisture and excess oil in the moving parts aggravate the operation of these parts or stop them completely. Grenade Launcher should be disassembled and cleaned thoroughly with a dry cloth. If necessary, dry cleaning should be done to remove excess oil. Parts showing signs of wear are wiped with a slightly dampened cloth using special oil.

- If possible, the grenade launcher is cleaned/maintained is a closed and warm place at room temperature.

- Lubricate inner surfaces, action parts like the bolt, springs and trigger group parts of the grenade launcher. On the other hand, external surface must be as dry as possible.

- When the grenade launcher is taken out of the warm place at low temperatures, it must be covered. Otherwise, a humidity layer will from on the grenade launcher due to rapid change of temperature, and this may lead to rusting.

- When the grenade launcher is carried from a cold environment to a warm place like a room or tent, it must be disassembled and dried.

- Keep the grenade launcher dry as far as possible, and apply function tests every 30 minutes due to freezing risk.

- Never leave hot guns on Ice or snow.

- Room temperature where the grenade launchers are stored should equal to outside temperature so as to avoid problems due to rapid change of temperature.

- When guns are transferred from a cold place to a hot one, they must be dehumidified and cleaned. Otherwise, frosting will occur on metal surfaces due to condensation.

c.	Hot and Humid Conditions:

- Clean and maintain the grenade launcher as necessary.

- Increase the frequency of inspection, maintenance and lubrication.

- Check interior of the grenade launcher regularly for rust and corrosion.

- Rifling of the barrel Is Important and should be taken care of. The bore must remain lightly lubricated when possible.

- Use the lubricants for maintenance more often.

- Store the grenade launcher in a closed case/box when possible.

- Avoid contact to sea/salty water. İn case of contact with salty water, disassemble

d.	Hot and Dry Air Conditions:

Dirt on/in the barrel and action parts must be daily cleaned. To prevent dirt from gathering at dirty areas, the grenade launcher must always remain dry. Muzzle, and mechanism must be protected from sand and dust storms by covering. After being exposed to dusty/sandy conditions, and sign of rust appears, the grenade launcher must be lubricated again.

- Regularly clean the grenade launcher.

- Avoid excessive lubrication as it will form oil residue on the gun.

- İnterior and action parts must be lubricated, yet outer surface should remain dry.

- Rust and corrosion should not be experienced at such condition, yet the guns should be checked from time to time.

- Store the grenade launcher in a closed case/box when possible.

- İnterior of the barrel must be cleaned from dust and sand using a cleaning rod and kept dry.

- To protect the gun from sand, it must be kept in a case or crate.

- Clean barrel and chamber every day.

- Try avoiding sand getting into the barrel.

- Drying and slightly lubricating the gun helps avoiding rust arising due to the sweating of hands and cheek.

ADVANCED LEVEL MAINTENANCE-REPAIR WARNING

- All screws in Grenade Launcher assembly are tightened with a torque of 1.0 Nm.

- Since all screws are used in the final assembly, Lock Tight (screw fixing adhesive) is used, if any of the screws need to be removed during maintenance and repair, they will be removed by heating method with a heater giving a temperature of approximately 300°C for 4-5 minutes.

- Muzzle side not directed to an unsafe target, the gun is placed on gun maintenance table or a flat surface. Avoid hitting the muzzle or sight system to the table.

- While cleaning interior of the barrel, cleaning must be done from chamber side to the muzzle, and muzzle should be hold lower than chamber side to prevent dirt and oil from reaching to loading/unloading and firing mechanisms.

- Remove any dirt from the cleaning rod before starting to dean the barrel.

- Insert the cleaning from chamber side and move it in direction of the muzzle. After cleaning rod goes out at the muzzle side, take the cloth out and remove the end piece. Next, pull the cleaning rod backward and complete the process.

- First, attach a dry and dean cloth (swab) to cleaning rod and apply a rough cleaning for the interior of barrel.

- Attach a cloth wet with maintenance oil and lubricate the interior of the barrel.

- Wait for about 15-20 minutes for the maintenance oil to soften any residue inside the barrel.

- Attach the provided bronze brush to the cleaning rod (Figure 49) and wet with oil.

- For every 3 shots, 1 cleaning with the bronze brush is necessary, and this ratio is valid cleaning.

- Next step of the cleaning goes on with the cotton brush (Figure 50).

- Attach a dry and dean cloth (swab) to the cleaning rod (provided with the cleaning/maintenance kit) and move the cleaning rod in the bore forward/backward until interior of the barrel becomes completely dean.

- To dean the chamber, attach the cleaning brush with Steel wire (thin or thick one as required), and brush it.

- Using the plastic wired cleaning brushes within the cleaning/maintenance kit, dust and dirt on outer surfaces on the gun can be cleaned.

WARNING! When cleaning the inside of the barrel with a slush brush and slush cloth, never pull the rod from the end of the barrel towards the firing chamber!

WARNING! Never use force when disassembling, cleaning and assembling the weapon!

STORAGE

    - Preparation for Storage: Before your firearm is stored, it should be thoroughly cleaned and lightly oiled. A thin layer of high-quality firearm oil should be applied to all exposed surfaces. In humid conditions, ensure that cleaning cloths used are dry. After the metal parts are dried, they should not be touched with bare hands. For normal storage periods, metal parts are lightly oiled with protective oil, and for longer periods, they are oiled with grease. The firearm should never be wrapped with cloth or similar material, and no plug should be placed in the barrel, as these can cause the firearm to rust by absorbing moisture.

    - Cleaning After Long-Term Storage: A firearm taken out of storage after an extended period should be thoroughly cleaned and oiled with normal protective oil. If it is to be fired immediately, ensure that the oil used for storage (protective or grease) is completely removed, the inside of the barrel is dry, and there is no obstruction before firing.

BEFORE STORING, CHECK IF YOUR SEMI-AUTOMATIC PISTOL IS LOADED. STORE FIREARMS AND AMMUNITION SEPARATELY AND IN A SECURE PLACE INACCESSIBLE TO UNAUTHORIZED INDIVIDUALS. ALWAYS LOCK YOUR FIREARMS SAFELY WHEN NOT IN USE. CONSULT YOUR COUNTRY'S FIREARMS REGULATIONS FOR THE SAFE STORAGE OF FIREARMS AND AMMUNITION.

WARNING! Modifying or changing parts and/or internal safeties is dangerous and voids the warranty. This firearm is designed to function properly with original parts. It is your responsibility to ensure that all purchased parts are intended for this firearm, installed correctly, and both spare and original parts are not replaced or modified. Your firearm is a complex and delicate tool with many parts that need to be correctly associated with other parts for proper and safe operation. Assembling a firearm with incorrect or modified parts can lead to damage, danger, and malfunction, resulting in injury or death to yourself and others. Always consult a qualified gunsmith for any work on your firearm.

| USER DAILY AND WEEKLY CARE FORM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PLACES TO BE MAINTAINED | DAILY CARE |  |  | WEEKLY CARE |
|  |  | SERVICE BEFORE | SERVICE DURING | SERVICE AFTER |  |
| 1 | OVERALL CONDITION Checking of the broken and inoperative part on outer surface of the gun. Missing screws and pins are completed. unsuitable/improper screws and pins are replaced. | X | X | X | X |
| 2 | BARREL and SHOOTING BED Checking the inside of the barrel and the firing chamber for foreign materials, scratches, broken sets and tingling. Cleaning and drying the inside of the barrel. | X | X | X | X |
| 3 | Barrel Opening Lever And Cocking System Checking that the barrel release latch pushes the barrel forward when pressed, and that it locks when the barrel is pulled backwards from the handguard after loading. | X |  | X | X |
| 4 | COCKING SHAFT AND FIRING ASSEMBLY |  |  |  |  |
|  | a. Check that the firing pin and cocking mechanism are in working order, clean and lightly oil the moving parts. | X |  | X | X |
|  | b. Cleaning and oiling of all parts of the trigger assembly and checking the operability of each part. | X |  | X | X |
| 5 | SIDE TURN FORMATION |  |  |  |  |
|  | a. Checking and controlling the operation of all parts that provide the side rotation movement and lubricating them. | X |  | X | X |
| 6 | SIGHT ASSEMBLY |  |  |  |  |
|  | a. Check the sight assembly connections for proper functioning and oil the pins. | X |  | X | X |
| 7 | CONNECTION SYSTEM |  |  |  |  |
|  | a. Checking the deformation and cleanliness of the Grenade Launcher Picatinny rail connection channel, checking the operation of the fixing bearings and screws. | X |  | X | X |
|  | b. Oiling of fixing bearings and screws. | X |  | X | X |
| 8 | STAND ALONE AND STOCK ASSEMBLY |  |  |  |  |
|  | a. Checking the deformation and cleanliness of the individual stock group front connection slide. | X |  | X | X |
|  | b. Checking the travelling slide of the telescopic stock. | X |  | X | X |
| 9 | BARREL LIFE SPAN Using a specific or multipurpose gauge, service life of the barrel is determined. |  |  |  |  |
| INSTRUCTION MANUAL |  |  |  |  |  |
| - This form is used by the grenadiers for daily and weekly cleaning/maintenance. - User of the grenade launcher and his commanding officer in charge are responsible for the proper application ol this form. - This form is used as a guide for daily and weekly guide, and not filled. - The form is also used in technical inspections and tests. - Work to be done are marked with X on the form. - Identified problems are reported to the commanding officer in charge. - Any test and maintenance to be done are based on the relevant technical field manual, and maintenance manuals. |  |  |  |  |  |

YEDEK PARÇA KATALOĞU

Exploded View

Part List